- ADEMCO VISTA-120IT

- Partitioned Security System

- With Scheduling

- Programming Guide

This control complies with EN50131

Table Of Contents

Recommended Programming Procedure	4

Program Field Index	5

VISTA 120 Programming Form	6

Partition-Specific Fields	11

#93 Menu Mode Programming	13

Zone Programming	14

Expert Mode Zone Programming	17

Report Code Programming	20

Alpha Descriptor Programming	23

ALPHA DESCRIPTOR VOCABULARY	27

Device Programming	28

Output Programming	29

Relay Voice Descriptors	34

Relay Voice Descriptors and Custom Word    Substitutes Vocabulary	35

Custom Word Substitutes for VIP Module    Annunciation	35

System Layout Worksheets	36

Output Devices Worksheets	48

Scheduling Menu Prompts	54

Scheduling Worksheets	55

VISTA-120IT Summary of Connections Diagram	63

The purpose of this document is to provide a quick and easy way to program your VISTA-120IT system.  A recommended programming procedure is included, followed by a list of program fields with the corresponding program group they belong to (System-wide, partition-specific, scheduling, etc.).

Following the program forms are system layout worksheets.  We recommend that you use these sheets to plan your system before programming is performed.  If you need further information about specific programming options, see the VISTA-120IT INSTALLATION INSTRUCTIONS.

- Single-Partition System

The system default is for a single-partition system.  Use the VISTA-120IT SINGLE PARTITION PROGRAMMING FORM when programming for single-partition usage.  Follow the steps outlined in recommended programming procedure of this document for proper programming procedure.

- Multiple-Partition System

You must enter the number of partitions you are using in data field 2*00 to set the system for multiple partitions. Use the VISTA-120IT SINGLE PARTITION and the PARTITION-SPECIFIC PROGRAM FORMS when programming the system for multiple partitions.  Follow the steps outlined in recommended programming procedure of this document for proper programming procedure.

Make sure that one keypad is connected to the control and is set to device address "00."

SUMMARY OF PROGRAMMING COMMANDS

•	To enter program mode, enter installer code + [8] + [0] + [0] + [0]

•	To set standard defaults, press 97

•	To set communication defaults, press 94 + one of the following: 80 = low speed; 81 = Ademco Express; 82 = Ademco Expanded High Speed; 83 = Ademco Contact ID

•	To change to next page of program fields, press 94

•	To return to previous set of fields, press 99

•	To erase account & phone number field entries, press [] + field number + []

•	To assign zone descriptors, press #93 + follow menu prompts

•	To add custom words, press #93 + follow menu prompts

•	To enter Installer's Message, press #93 + follow menu prompts

•	To exit program mode, press 99 to allow reaccess to programming mode by installer code, or press 98 to prevent reaccess to programming mode by installer code.

Factory-set default values (97) are shown in brackets [  ].

## Recommended Programming Procedure

The following is a step-by-step procedure recommended for programming your VISTA-120IT system.

1.	Set the keypads (and other peripheral devices) to the appropriate addresses.

2.	Set factory defaults by pressing 97.

This will automatically enable keypad addresses 00-03, so be sure at least one keypad is set to one of these addresses.

3.	Program System-wide (global) data fields.

Using the programming form as a guide, enter program mode and program all system-wide programming fields (the ones that are not shaded in grey).  These options affect the entire system, regardless of partitions. They include control options, downloader and dialler options, RF options, event logging options, etc.

Note that field 200 (number of partitions) & field 132 (RF expander type) must be programmed before continuing.

4.	Program partition-specific fields.

Partition-specific fields can have different values for each partition, (they are the ones shaded in grey in the programming form). After the system-wide fields have been programmed, program all partition-specific programming fields by first pressing 91 to select a partition (while still in data field program mode). Then enter the first partition-specific field number 03. The next partition-specific field will automatically be displayed when you are finished entering the value for field 03.  To program the fields for the next partition, press 91, enter the desired partition number, then enter field 03.

5.	Use #93 Menu Mode for device programming.

Refer to the Device Programming section of this Programming Guide to assign keypad ID numbers and default partitions for each keypad, and to selectively suppress certain keypad sounding options.  Also use this mode to assign RF receivers and relay modules.

6.	Use #93 Menu Mode for zone programming.

Refer to the Zone Programming section of this Programming Guide to program zone response types, assign right loop zones and wireless zones, assign zones to partitions, and to program alarm report codes.

7.	Use #93 Menu Mode for programming relays.

Refer to the Relay Programming section of this Programming Guide to program desired relay operation.

8.	Program Communication options.

Refer to the COMMUNICATION PROGRAMMING section of the Installation Instructions to load communication defaults and to program related fields.  Then use #93 mode in this Programming Guide to program report codes if necessary.

9.	Use #93 Menu Mode for programming alpha descriptors.

Refer to the Alpha Descriptor Programming section of this Programming Guide to enter zone and partition descriptors and a custom installer's message.

10.	Use #93 Menu Mode for programming relay voice descriptors and custom word substitutes.

Refer to the Relay Voice Descriptors section of this Programming Guide for further instructions for programming relay descriptors to be annunciated by the 4286 VIP module*, as well as the CUSTOM INDEX section for custom word substitutes.

*Not applicable to TeleCommand usage.

11.	Use #80 Mode for programming schedules.

Refer to the SCHEDULING section of the Installation Instructions to program open/close schedules, temporary and holiday schedules, limitation of access schedules, and time driven events.

12.	Define user access codes.

Refer to SECURITY ACCESS CODES section of the Installation Instructions to program authority level, O/C reporting option, partition assignments, and wireless key assignments for each user.

13.	Exit Programming Mode

Exit programming mode by pressing either 98 or 99. A second entry of 99 is required if the exit is being done from fields 100 and above.

To prevent re-access to Programming mode using the Installer's code, use 98. The only way to re-access Programming mode is by depressing both the [] and [#] keys at the same time within 30 seconds of power up.

Exiting by using 99 always allows reentry into Programming mode using the Installer's code. Either way of exiting will allow access via downloading. Note that if local programming lockout is set via downloading, programming mode cannot be entered at the keypad.

- Program Field Index

On the following pages, the programming fields have been arranged in numerical order.  Use this index to cross-reference the fields on the programming form.

Field	Group

Field	Group

Field	Group

00	System-wide

01	System-wide

03	System-wide

04	System-wide

05	System-wide

06	System-wide

07	System-wide

08	System-wide

09	Partition-Specific

10	Partition-Specific

11	Partition-Specific

12	Partition-Specific

13	Partition-Specific

14	System-wide

15	System-wide

16	Partition-Specific

17	System-wide

18	System-wide

19	System-wide

20	System-wide

21	System-wide

22	Partition-Specific

23	Partition-Specific

24	System-wide

25	System-wide

26	Communications

27	Communications

28	System-wide

29	Partition-Specific

30	Communications

31	Communications

32	Partition-Specific

33	Communications

34	Communications

35	System-wide

36	System-wide

37	System-wide

38	Partition-Specific

39	Partition-Specific

40	Communications

41	System-wide

42	Communications

43	Communications

44	Communications

45	Communications

46	Communications

47	Communications

48	Communications

49	Communications

50	Communications

51	Communications

52	Communications

53	Communications

54	Communications

55	Communications

56	Communications

57	Communications

58	Communications

59	Communications

60	Communications

61	Communications

79	Communications

80	Communications

83	Communications

84	Partition-Specific

85	Partition-Specific

86	Communications

87	Partition-Specific

88	Partition-Specific

89	Communications

90	Partition-Specific

100	System-wide

101	System-wide

102	System-wide

105	System-wide

106	System-wide

107	System-wide

108	System-wide

109	System-wide

110	System-wide

111	System-wide

112	Partition-Specific

113	Partition-Specific

114	System-wide

115	Partition-Specific

116	Partition-Specific

117	System-wide

118	Partition-Specific

119	Partition-Specific

120	System-wide

121	System-wide

122	System-wide

123	System-wide

124	System-wide

125	System-wide

126	Partition-Specific

127	System-wide

128	System-wide

129	System-wide

130	System-wide

131	System-wide

133	Communications

134	Communications

135	System-wide

136	System-wide

137	System-wide

138	System-wide

139	System-wide

140	System-wide

141	System-wide

142	Communications

143	Partition-Specific

144	System-wide

145	Partition-Specific

146	System-wide

147	Partition-Specific

148	System-wide

149	System-wide

150	System-wide

151	System-wide

152	Partition-Specific

153	Communications

154	System-wide

155	System-wide

156	System-wide

157	System-wide

158	System-wide

159	System-wide

160	System-wide

161	System-wide

162	System-wide

166	System-wide

167	Communications

170	System-wide

171	System-wide

172	System-wide

173	System-wide

174	System-wide

175	System-wide

176	Partition-Specific

177	System-wide

200	System-wide

201	System-wide

202	System-wide

205	Partition-Specific

206	Partition-Specific

207	Partition-Specific

208	Partition-Specific

209	Partition-Specific

210	Partition-Specific

211	System-wide

218	Partition-Specific

219	System-wide

220	System-wide

221	Communications

222	Partition-Specific

223	Partition-Specific

224	Partition-Specific

225	System-wide

## VISTA 120 Programming Form

Some fields are programmed for each partition (shown as shaded fields). If you are programming a multiple-partition system, see the Partition-Specific Fields section for programming these fields. Standard default (*97) values are shown in brackets [ ]; otherwise, default = 0.

00	INSTALLER CODE	[4140]

Enter 4 digits, 0-9

01	MASTER TO ENABLE INSTALLER	[1]

(Requires User and Installer Code Entry to enter Program Mode)

1 = enable (Norwegian requirement); 0 = disable

03	FINAL CONTACT SET	[0]

1 = enable; 0 = disable

04	AUTOBYPASS EXIT ROUTE FAULTS WITH 	[0]

2ND ARMING ATTEMPT IN 5 SECS.

1 = enable (Swedish Requirement); 0 = disable

05	ARM WITH LOW BATTERY	[1]

1 = enable, 0 = disable (ANPI requirement)

06	ZONE TYPE 5 ALWAYS ALARM	 [0]

1 = enable; 0 = disable

07	ALLOW ARMING WITH FAULTS	[0]

IN EXIT ROUTE

1 = enable; 0 = disable

08	SELF ACTIVATING SIREN OUTPUT	[0]

1 = enable (ANPI requirement); 0 = disable

09	ENTRY DELAY #1	[02]

00 = no delay; 01-15 times 15 seconds

10	EXIT DELAY #1	[03]

00 = no delay; 01-15 times 15 seconds

11	ENTRY DELAY #2 	[03]

00 = no delay; 01-15 times 15 seconds (must be

longer than Entry Delay #1).

12	EXIT DELAY #2	[08]

00 = no delay; 01-15 times 15 seconds (must be

longer than Exit Delay #1).

13	ALARM SOUNDER TIMEOUT	[02]

01-15 multiplied by 1 minute.

14	ZONE 9 RESPONSE TIME	[0]

0 = normal response (350mec);  1 = fast response (10msec)

15	KEYSWITCH ASSIGNMENT	[0]

Enter partition in which keyswitch is used (1-8);

0 = disable keyswitch

16	CONFIRMATION OF ARMING DING	[0]

1 = enable; 0 = disable.

17	AC MAINS LOSS KEYPAD SOUNDING	[0]

1 = enable; 0 = disable.

18	MAINS PRESENCE DISPLAY	[0]

1 = enable; 0 = disable.

19	RANDOMISE AC MAINS LOSS REPORT	[0]

0 = normal report (approx. 2 min after AC loss)

1 = randomises reporting between 30-60 min;

20	4286 PHONE CODE

Enter 01 - 09 for first digit;	[00],	[11]

 or # = second digit of access code

(enter # +11 for “”, or # +12 for “#”) To disable enter 00 for 	the 1st digit

21	PREVENT FIRE ALARM TIMEOUT	[0]

1 = No timeout; 0 = Timeout (same as burg.).

22	KEYPAD PANIC ENABLES	[0][0][1]

1 = enable; 0 = disable 	995	996	999

23	MULTIPLE ALARMS 	[1]

1 = enable; 0 = disable

24	IGNORE EXPANSION ZONE TAMPER 	 [0]

0 = disable (tamper detection for RF and RPMs)

1 = enable (no tamper detection)

25	BURG.TRIG. FOR RESPONSE TYPE 8	[1]

1 = enable; 0 = disable

26	INTELLIGENT TEST REPORTING	[0]

1 = yes (no report sent if any other report was recently sent)

0 = no (send report at programmed interval, field *27)

27	TEST REPORT INTERVAL  	[024]

Enter interval in hours, 001-999

000 = no report;

28	POWER UP IN PREVIOUS STATE	[1]

1 = enable; 0 = disable

29	QUICK ARM   	[1]

1 = enable; 0 = disable

30	MULTIFREQUENCY OR

DECADIC PULSE DIAL	[0]

1 = multifrequency; 0 = decadic

31	PABX ACCESS CODE

00-09; B-F (11-15)

32	PRIM. SUBS. ACCT #

Enter 00-09; 	[15]	[15]	[15]	[15]

B-F (11-15)

33	PRIMARY PHONE NUMBER

Enter 0-9 for each digit.  Enter # + 11 for *; # + 12 for #; 	# + 13 for 2 second pause.  Do not fill unused spaces.

34	SECONDARY PHONE NUMBER

Enter 0-9 for each digit.  Enter # + 11 for *, # + 12 for #, 	# + 13 for 2 second pause.  Do not fill unused spaces.

35	DOWNLOAD PHONE NO.

Enter 0-9 for each digit.  Enter # + 11 for *, # + 12 for #, 	# + 13 for 2 second pause.  Do not fill unused spaces.

36	DOWNLOAD ID No.

[15]	[15]	[15]	[15]	[15]	[15]	[15]	[15]

- Enter 00-09; A-F (10-15)

37	DOWNLOAD COMMAND ENABLES

1 = enable; 0 = disable

† Restrict Download Access When Armed: Can only arm unarmed partitions, upload the program/event log, command relays, and request status. Default = 11011111.

38	PREVENT ZONE 	[000]

XXX BYPASS

Enter a zone number (001-128)

Enter 000 if all zones can be bypassed.

39	ENABLE OPEN/CLOSE	[1]

REPORT FOR INSTALLER CODE

1 = enable; 0 = disable

40	OPEN/CLOSE REPORT	[0]

FOR KEYSWITCH

1 = enable; 0 = disable

41	NORMALLY CLOSED or EOLR	[0]

(Zones 2 - 8)

1 = N.C. loops; 0 = EOLR supervision

42	SUPPRESS FIRE RELAY	[0]

1 = suppress fire alarm relay on 4204/Powerline Carrier Device relays

0 = 4204/Powerline Carrier Device fire alarm relay activates on fire alarms

43	SUPPRESS WIRELESS SIREN	[0]

ACTIVATION FOR FIRE ALARMS

1 = enable (no activation of wireless siren for fire alarms)

0 = disable (wireless sirens will be activated for fire alarms)

44	RING DETECTION COUNT	[00]

Enter 00 to disable ring detection.

Enter 01-14 for ring counts of 1-14.

Enter 15 to select Answering/Fax Machine Defeat Mode

45	PRIMARY FORMAT	[1]

0 = Low Speed;	2 = Ademco Exp. High Speed;

1 = Contact ID; 	3 = Ademco Express

46	LOW SPEED FORMAT (Primary)	[0]

0 = ADEMCO Low Speed; 1 = Sescoa/Radionics

47	SECONDARY FORMAT	[1]

0 = Low Speed;	2 = Ademco Exp. High Speed;

1 = Contact ID; 	3 = Ademco Express

48	LOW SPEED FORMAT (Secondary)	[0]

0 = ADEMCO Low Speed; 1 = Sescoa/Radionics

49	CHECKSUM VERIFICATION	 [0] [0]

0 = disable; 1 = enable	Prim	Sec

50	SESCOA/RADIONICS SELECT	[0]

0 = Sescoa/	1 = Radionics

51	DUAL REPORTING	[0]

0 = disable; 1 = enable

If used with Spilt Reporting "1" option (1*34), alarms and alarm restores go to both primary & secondary numbers, while all other reports go to secondary only.  If used with Split Reporting "2" option, alarms, alarm restores and recent close go to both, open/close and test messages go to secondary only, while all other reports go to primary only. If used with Split Reporting ‘3’ option, alarms and restores go to both, all others go to the secondary only.

52	STANDARD/EXPANDED REPORTING PRIMARY

[0 0 0 0 0 0]

0 = standard; 1 = expanded;

NOTE: Expanded overrides choices in 4+2 format.

53	STANDARD/EXPANDED REPORTING SECONDARY

[0 0 0 0 0 0]

0 = standard; 1 = expanded;

Note: Expanded overrides choices in 4+2 format.

54	MAX. No. OF DIALER ATTEMPTS	[8]

Enter 1 - 8

55	COUNTRY CODE	[00]

00 = Latin America, Spain, Italy, Eastern Europe, China

01 = Australia; 05 = France; 02 = Belgium; 06 = Netherlands;

03 = Denmark; 07 = Norway; 04 = Finland; 08 = Sweden

Note: Options 01-08 require special hardware configuration

56	DYNAMIC SIGNALING DELAY	[00]

Select the delay time (00-15) times 15 seconds before sending to second destination.

57	DYNAMIC SIGNALING PRIORITY	[0]

0=Primary dialer; 1=LRR, as first reporting destination.

58	LRR CENTRAL STATION #1 CATEGORY ENABLE

[000000]

0 = disable; 1 = enable

59	LRR CENTRAL STATION #2 CATEGORY ENABLE

[000000]

0 = disable; 1 = enable

60	VERIFIED ALARM REPORT ENABLE	[0]

0 = disable; 1 = enable (Swedish requirement)

61	ROBOFON VERSION OF CONTACT ID	[0]

0 = disable; 1 = enable (Swedish requirement)

79	RESTORE ENABLES FOR ZONE TYPES 1-8

[0 = disable], 1 = enable

80	RESTORE ENABLES FOR ZONE TYPES 9 AND 10

[0 = disable]; 1 = enable

83	FIRST TEST REPORT TIME

Day	Hour	Min

Enter 00-07 for the day (01 = Monday).  Enter 00-23 for the hour.  Enter 00-59 for the minutes.  Enter 00 in all locations to enable Test Report to be sent immediately. [00 12 00]

84	INTERMITTENT SENSOR	[03]

SUPPRESSION

Enter 01-15. Enter 00 for unlimited reports

85	ENABLE DIALER REPORTS FOR PANICS & DURESS

1=enable; [0=disable]

86	REPORT/LOG ZONE TYPE 23	[0]

0 = disable; 1 = enable

87	ENTRY WARNING	[1]

0 = 3 short beeps; 1 = slow continuous beeps

88	BURG. ALARM COMM. DELAY	[1]

0 = no delay; 1 = 30-second delay

89	RESTORE REPORT TIMING	[0]

0 = instant; 1 = after siren timeout if zone is restored

2 = when system is disarmed.

90	SEC. SUBS. ACCT.#

Enter 00-09; B-F (11-15)    [15 15 15 15]

2nd Page Programming Fields (press *94)

100	CONTACT ID REPORTING IN	[0]

ASCII THROUGH PRINTER PORT

0 = printer – no ISDN; 1 = ISDN + callout; 2 = ISDN only;      3 = callout if ISDN failed

101	ASCII CONTACT ID REPORTING	[0]

WITH OR WITHOUT ACK.

0 = ACK required; 1 = ACK not required

102	ASCII CONTACT ID BAUD RATE	[0]

0 = 1200; 1 = 2400; 2 = 4800

105	BYPASS ENABLE FOR FIRE ZONES	[0]

0 = fire zones cannot be bypassed

1 = permits the bypassing of fire zones

106	SUPPRESS ALL KEYPAD DISPLAYS	[0]

0 = disable; 1 = enable

107	CHECK OR TRBL DISPLAY	[0]

0 = CHECK; 1 = TRBL

108	SUPPRESS USE OF "ARMED" 	[0]

LED ON KEYPADS

0= disable; 1= enable.

(For countries where Red is only for alarm)

109	SUPPRESS KEYPAD ARMING STATUS	[1]

INDICATIONS WHEN SYSTEM IS ARMED

0 = disable; 1 = enable

110	FIRST TO ALARM DISPLAY LOCK	[0]

0 = scroll alarms; 1 = lock display

111	COMMON AREA 1 PARTITION	[0]

Enter 1-8; 0 = none

112	AFFECTS COMMON AREA 1	[0]

0 = disable; 	1 = enable, if this partition affects common area 1

113	ARMS COMMON AREA 1		[0]

0 = disable; 1 = enable, if this partition attempts to arm common area 1

114	COMMON AREA 2 PARTITION	[0]

Enter 1-8; 0 = none

115	AFFECTS COMMON AREA 2 	[0]

0 = disable; 1 = enable, if this partition affects common area 2

116	ARMS COMMON AREA 2	[0]

0 = disable;  1 = enable, if this partition attempts to arm common area 2

117	COMMON AREA 3 PARTITION	[0]

Enter 1-8; 0 = none

118	AFFECTS COMMON AREA 3 	[0]

0 = disable; 	1 = enable, if this partition affects common area 3

119	ARMS COMMON AREA 3 	[0]

0 = disable; 1 = enable, if this partition attempts to arm common area 3

120	AUTOBYPASS FAULTED 	[0]

EXIT ROUTE ZONES (Australian Feature)

0 = disable; 1 = enable

121	EXIT DELAY RESET 	[0]

0 = disable; 1 = enable

FIELDS 122-125:  Allow four sets of two zones each to be linked so that both must fault within a five minute period to cause an alarm. Default for these fields = [000], [000].

1*22	CROSS-ZONING PAIR ONE

1*23	CROSS-ZONING PAIR TWO

1*24	CROSS-ZONING PAIR THREE

1*25	CROSS-ZONING PAIR FOUR

126	PANIC BUTTON OR SPEED KEY

[00, 00, 00, 00]

00 = panic function (for D key = not used)

01-32 = macro number

127	FIELD 1*31 XMTR CHECK-IN 	[0]

SUPERVISION INTERVAL TO BE MULTIPLE OF 1-HOUR INSTEAD OF 2 HOURS

0 = 2 hours; 1 = 1 hour (must be 1 hour for CENELEC Grade 2 compliance)

128	RF XMTR LOW BATTERY SOUND	[0]

0 = disarmed state only; 	1 = both armed and disarmed states

129	RF XMTR LOW BATTERY REPORTING	[0]

0= disable; 1= enable

130	RF RCVR CHECK-IN INTERVAL	[06]

02-15 times 2 hours; 00 disables supervision

131	RF XMTR CHECK-IN INTERVAL	[12]

02-15 times 2 hours;  00 disables transmitter supervision

(Select 02 for CENELEC Grade 2 compliance)

132	ACCESS CONTROL DIALER ENABLE

[0 0 0 0 0 0]

133	MULTIFREQUENCY DIALING 	[0]

WITH DECADIC PULSE DIAL BACKUP

0= disable; 1= enable

134	COMM. SPLIT REPORT SELECTION 	[0]

0=no; 1=alarms and alarm restores primary, others secondary; 2=open/close, test secondary, others primary; 3=fire alarm and fire restores both, others secondary.  See T51 for comments if using with dual reporting.

135	LOW BATTERY TEST INTERVAL 	[0]

0 = 13 second test every 4 minutes (ANPI requirement)

1 = 1.5 second test every 50 seconds (Norwegian req.)

136 	CPU FAIL TRIGGER OUTPUT	[0]

0 = disable (normal use for Output 2); 1 = enable (Output trigger 2 on J7 to be CPU fail output, overriding any other selection for Output 2) (CENELEC requirement)

137	TLM INPUT ON ZONE 9	[0]

0 = disable (normal use for zone 9);  1 = enable (telephone line fault monitor output to be fed into zone 9)

138	USER OVERRIDE OF TAMPER ALARMS 	[1]

INSTEAD OF INSTALLER ONLY RESET

0 = disable (ANPI requirement); 1 = enable

139	USER OVERRIDE OF TAMPER FAULTS	[1]

INSTEAD OF INSTALLER ONLY BYPASS

0 = disable (ANPI requirement); 1 =enable

140	MAX. NUMBER OF ZONES THAT	[00]

CAN BE BYPASSED PER PARTITION

Enter 01-15 (ANPI requirement); 00 = no restriction

141	BYPASS/UNBYPASS ZONES 	[0]

WHEN ARMED

0 = disable; 1 = enable

142	CALL WAITING DEFEAT	[0]

0 = disable; 1 = enable

143	PERMANENT. KEYPAD BACKLIGHT	[0]

0 = disable (display lights when any key is pressed, and turns off after keypad inactivity); 1 = enable

144	KEYPAD TAMPER DETECT	[1]

0 = disable; 1 = enable

145	EXIT DELAY SOUNDING	[0]

0 = disable; 1 = enable (slow beeping then changes to rapid beeping during last 10 sec of exit delay)

146	AUXILIARY OUTPUT MODE	[0]

0 = ground start output;

1 = smoke detector reset

2 = keypad-like sounding at an auxiliary sounder

3 = AAV module is being used

For option 2, a partition must be selected in field 15

147	CHIME ON EXTERNAL SIREN	[0]

0 = disable; 1 = enable

148	WIRELESS KEYPAD ASSIGNMENT	[0]

0=disable; enter partition in which RF keypad used, 1-8

149	SUPPRESS TX SUPERVISION FAIL SOUND	[1]

0 = disable; 1 = enable

150	NO. SECONDS ADDED PER DAY	[00]

00-30 = number of seconds needed to be added per day to correct the real-time clock. (Only use if 1*54 = 1)

151	NO. SECONDS REMOVED PER DAY	[00]

00-30 = number of seconds needed to be removed per day to correct the real-time clock. (Only use if 1*54 = 1)

152	SEND CANCEL IF ALARM + OFF	[0]

0 = disable (Cancel reports are sent within Alarm Sounder Timeout period only); 1 = enable (no restriction)

153	DISABLE DOWNLOAD CALLBACK	[0]

0 = callback required; 1 = callback not required

154	INTERNAL CLOCK SYNC 	[0]

0 = use AC sync for clock; 1 = use internal crystal for real-time clock;

155	INTERNATIONAL DATE FORMAT	[1]

0 = MM/DD/YY; 1 = DD/MM/YY

156	AC 60Hz/50Hz	[1]

0 = 60Hz; 1 = 50Hz

157	5800 RF BUTTON GLOBAL ARM	[0]

0 = disable; 1 = enable

158	5800 RF BUTTON FORCE ARM	[0]

0 = disable; 1 = enable  (If a zone is faulted after pressing button, keypad will beep once. User should press button again within 4 sec. to force-bypass those zones.)

159	SUPPRESS STATUS LED OUTPUT WHEN	[0]

ZONE 7 KEYSWITCH ENABLED/RETAIN VOLTAGE

TRIGGER. OUTPUTS

0 = disable; 1 = enable

160	ZONE 5 AUDIO Alarm Verification	[0]

0 = disable; 1 = enable zone 5 to be used for 2-way AAV

161	DISPLAY TAMPER	[0]

Enter 1 to display "TAMPR" instead of "CHECK" or "TRBL" (see field 1*07).

162	TAMPER DETECT IN TEST MODE	[0]

Enter 1 to terminate Test mode upon tamper condition; Enter 0 to ignore tamper conditions during Test mode (displays "FAULT")

166	SILENCE SOUNDERS DURING AAV	[0]

0 = disable; 1 = enable (AAV in use)

167	VIDEO ALARM VERIFICATION	[0]

0 = disable; 1 = enable

170	EVENT LOG TYPES

[1 1 1 1 1]

1=enable; 0=disable

171	12/24 HOUR FORMAT TIME STAMP FORMAT

[0]

0 = 12 hour; 1 = 24 hour;

172	EVENT LOG PRINTER ON-LINE	[0]

0 = disable; 1 = enable;

173	PRINTER BAUD RATE	[0]

0 = 1200 (preferred); 1 = 300

174	RELAY TIMEOUT XXX MINUTES	[000]

Enter 000 – 127 times 2 minutes (000 - 254)

175	RELAY TIMEOUT YYY SECONDS	[000]

Enter 000 – 127

176	ACCESS CONTROL RELAY 	[00]

00 = relay not used; 01 – 96 (enables assigned relay to close for 2-seconds when the user enters the security code + [0])

177	LOG FIRST MAINTENANCE SIGNAL 	[0]

0 = no logging; 1 = log first maintenance signal;

3rd Page Programming Fields (press *94)

200	NUMBER OF PARTITIONS 	[1]

Enter the number of partitions used in system, 1 – 8

201	SUMMER TIME 	[04]  	[10]

START/END MONTH	Start	End

00-12. If summer time does not apply, enter 00, 00

202 	SUMMER TIME	[1, 5]

START/END WEEKEND #	Start|End

Enter 1-7. 1 = first; 2 = second; 3 = third; 4 = fourth; 5 = last; 6 = next to last; 7 = 3rd from last; 	Default: [5,5] = Last Sunday in April, last in Oct.]

205	AUTO-ARM DELAY 	[15]

15 = no auto arming; 00 = no delay; 01-14 times 4 minutes (04-56) delay; Enter the time between the end of the arming time period and the start of auto-arming warning period.

206	AUTO-ARM WARNING PERIOD 	[15]

00 = no warning period; 01-15 times 1-minute warning. This is the time during which the user is warned to exit the premises prior to the auto-arming of the system (beeps every 15 seconds, “ALERT” displayed).

207	AUTO-DISARM DELAY 	[15]

15 = no auto disarming; 00 = no delay; 	01-14 times 4 minutes (04-56) delay. This is the time between the end of the disarming time period and the start of auto-disarming.

208	ENABLE FORCE ARM FOR AUTO-ARM	[0]

0 = disable; 1 = enable

209	OPEN/CLOSE REPORTS BY EXCEPTION	[0]

0 = disable; 1 = enable. If enabled, only openings and closings occurring outside the scheduled opening/closing time periods will trigger dialler reports. Opening reports will also be suppressed during the closing time period in order to prevent false reports, when the user arms the system and then reenters the premises to retrieve a forgotten item.

210	ALLOW DISARMING ONLY DURING 	[0]

ARMING/DISARMING WINDOWS

0 = disable; 1 = enable

This feature adds high security to the installation.

211	ALLOW DISARM OUTSIDE WINDOW 	[0]

IF ALARM OCCURS

0 = disable; 1 = enable. Used only if field 210 (partition-specific field) is set to "1". If this field is enabled ("1") the system can be disarmed outside the disarm time period if an alarm has occurred. If "0", disarming can only be done during the disarm time period. If field 210 is set to "0" for a partition, this field has no effect for that partition.

218 	ENABLE GOTO FOR THIS PARTITION	[0]

0 = disable; 1 = enable. If enabled, this partition can be accessed from another partition’s keypad using the GOTO command.

219 	USE PARTITION DESCRIPTORS 	[0]

1 = enable; 0 = disable

220	ENABLE J7 TRIGGERS FOR PART.	[1]

0 = disable; 1 = enable

221	ENABLE SUPERVISION PULSES FOR LRR TRIGGER OUTPUTS	[000]

Enter 0 to disable or 1 to enable the listed outputs.

F= Fire; B= Burglary; P= Silent Panic/Duress.  Used for supervised connection to a transmitter.

222	DISPLAY FIRE ALARMS OF OTHER	[0]

PARTITIONS

0 = disable; 1 = enable

223	DISPLAY BURG & PANIC ALARMS	[0]

OF OTHER PARTITIONS

0 = disable; 1 = enable

224	DISPLAY TROUBLES OF OTHER 	[0]

PARTITIONS

0 = disable; 1 = enable

225 	OVERRIDE AC/COMM FAIL 	[1]

0 = disable: 1 = enable

- Partition-Specific Fields

(Duplicate this page for each partition in the installation.)

- To program these fields,

1.	Press *91 to select a partition.

2.	Enter a partition-specific field number (ex. *09).

3.	Make the required entry.

4.	Repeat steps 1-3 for each partition in the system.

- PARTITION #____ PROGRAM FIELDS

- 1st Page Fields

09	ENTRY DELAY #1	[02]

00 = no delay; 01-15 times 15 seconds

10	EXIT DELAY #1	[03]

00 = no delay; 01-15 times 15 seconds

11	ENTRY DELAY #2 	[03]

00 = no delay; 01-15 times 15 seconds (must be

longer than Entry Delay #1).

12	EXIT DELAY #2	[08]

00 = no delay; 01-15 times 15 seconds (must be

longer than Exit Delay #1).

13	ALARM SOUNDER TIMEOUT	[02]

01-15 multiplied by 1 minute.

16	CONFIRMATION OF ARMING DING	[0]

1 = enable; 0 = disable.

22	KEYPAD PANIC ENABLES	[0][0][1]

1 = enable; 0 = disable 	995	996	999

23	MULTIPLE ALARMS 	[1]

1 = enable; 0 = disable

29	QUICK ARM   	[1]

1 = enable; 0 = disable

32	PRIM. SUBS. ACCT #

Enter 00-09; 	[15]	[15]	[15]	[15]

B-F (11-15)

38	PREVENT ZONE 	[000]

XXX BYPASS

Enter a zone number (001-128)

Enter 000 if all zones can be bypassed.

39	ENABLE OPEN/CLOSE	[1]

REPORT FOR INSTALLER CODE

1 = enable; 0 = disable

84	INTERMITTENT SENSOR	[03]

SUPPRESSION

Enter 01-15. Enter 00 for unlimited reports

85	ENABLE DIALER REPORTS FOR PANICS & DURESS

1=enable; [0=disable]

87	ENTRY WARNING	[1]

0 = 3 short beeps; 1 = slow continuous beeps

88	BURG. ALARM COMM. DELAY	[0]

0 = no delay; 1 = 30-second delay

90	SEC. SUBS. ACCT.#

Enter 00-09; B-F (11-15)    [15 15 15 15]

2nd Page Programming Fields (press *94)

112	AFFECTS COMMON AREA 1	[0]

0 = disable; 	1 = enable, if this partition affects common area 1

113	ARMS COMMON AREA 1		[0]

0 = disable; 1 = enable, if this partition attempts to arm  common area 1

115	AFFECTS COMMON AREA 2 	[0]

0 = disable; 1 = enable, if this partition affects common area 2

116	ARMS COMMON AREA 2	[0]

0 = disable;  1 = enable, if this partition attempts to arm common area 2

118	AFFECTS COMMON AREA 3 	[0]

0 = disable; 	1 = enable, if this partition affects common area 3

119	ARMS COMMON AREA 3 	[0]

0 = disable; 1 = enable, if this partition attempts to arm common area 3

126	PANIC BUTTON OR SPEED KEY

[00, 00, 00, 00]

00 = panic function (for D key = not used)

01-32 = macro number

143	PERMANENT. KEYPAD BACKLIGHT	[0]

0 = disable (display lights when any key is pressed, and turns off after keypad inactivity); 1 = enable

145	EXIT DELAY SOUNDING	[0]

0 = disable; 1 = enable (slow beeping then changes to rapid beeping during last 10 sec of exit delay)

147	CHIME ON EXTERNAL SIREN	[0]

0 = disable; 1 = enable

152	SEND CANCEL IF ALARM + OFF	[0]

0 = disable (Cancel reports are sent within Alarm Sounder Timeout period only); 1 = enable (no restriction)

176	ACCESS CONTROL RELAY 	[00]

00 = relay not used; 01 – 96 (enables assigned relay to close for 2-seconds when the user enters the security code + [0])

3rd Page Programming Fields (press *94)

205	AUTO-ARM DELAY 	[15]

15 = no auto arming; 00 = no delay; 01-14 times 4 minutes (04-56) delay; Enter the time between the end of the arming time period and the start of auto-arming warning period.

206	AUTO-ARM WARNING PERIOD 	[15]

00 = no warning period; 01-15 times 1-minute warning. This is the time during which the user is warned to exit the premises prior to the auto-arming of the system (beeps every 15 seconds, “ALERT” displayed).

207	AUTO-DISARM DELAY 	[15]

15 = no auto disarming; 00 = no delay; 	01-14 times 4 minutes (04-56) delay. This is the time between the end of the disarming time period and the start of auto-disarming.

208	ENABLE FORCE ARM FOR AUTO-ARM	[0]

0 = disable; 1 = enable

209	OPEN/CLOSE REPORTS BY EXCEPTION	[0]

0 = disable; 1 = enable. If enabled, only openings and closings occurring outside the scheduled opening/closing time periods will trigger dialler reports. Opening reports will also be suppressed during the closing time period in order to prevent false reports, when the user arms the system and then reenters the premises to retrieve a forgotten item.

210	ALLOW DISARMING ONLY DURING 	[0]

ARMING/DISARMING WINDOWS

0 = disable; 1 = enable

This feature adds high security to the installation.

218 	ENABLE GOTO FOR THIS PARTITION	[0]

0 = disable; 1 = enable. If enabled, this partition can be accessed from another partition’s keypad using the GOTO command.

220	ENABLE J7 TRIGGERS FOR PART.	[1]

0 = disable; 1 = enable

222	DISPLAY FIRE ALARMS OF OTHER	[0]

PARTITIONS

0 = disable; 1 = enable

223	DISPLAY BURG & PANIC ALARMS	[0]

OF OTHER PARTITIONS

0 = disable; 1 = enable

224	DISPLAY TROUBLES OF OTHER 	[0]

PARTITIONS

0 = disable; 1 = enable

## #93 MENU MODE;#93 Menu Mode Programming

NOTE: Be sure that field 200: NUMBER OF PARTITIONS is programmed before proceeding. Additionally, program receivers by referring to the Device Programming section in this manual.

After programming all system wide related programming fields, press #93 while still in programming mode. Press 0 (NO) or 1 (YES) in response to the displayed menu selection. Pressing 0 displays the next choice in sequence.

#93 MENU MODE KEY COMMANDS

The following is a list of commands used while in the menu mode.

| #93 | Enters Menu Mode. |
| --- | --- |
| [] | Serves as [ENTER] key. Press to have keypad accept entry. |
| [#] | Backs up to previous screen. |
| 0 | Press to answer NO. |
| 1 | Press to answer YES. |
| 00, or 000+[] | Quits the Menu Mode and goes back into Data Field Programming Mode, if entered at the first prompt of each main menu option. |

Menu selections are as follows:

| PROMPT | EXPLANATION |
| --- | --- |
| ZONE PROG? 1 = YES 0 = NO 0 | For programming the following: - Zone Number - Zone Response Type - Partition Number for Zone - Dialer report code for zone - Input Device Type for zone (whether RF, polling loop, etc.) - Enrolling serial numbers of 5800 Series transmitters & serial polling loop devices into the system. |
| EXPERT MODE? 1 = YES 0 = NO 0 | Same as Zone Programming except: - Done with a minimum number of keystrokes. - Can program wireless keys using pre-defined templates. NOTE: Some of the zone attributes cannot be programmed in the Expert Mode, only in Zone Programming. |
| REPORT CODE PROG? 1 = YES 0 = NO 0 | For programming the following: - Alarm report codes for zones - Restore and supervisory codes - All other system report codes |
| ALPHA PROG? 1 = YES 0 = NO 0 | For entering alpha descriptors for the following: - Zone Descriptors - Installer’s Message - Custom Words - Partition Descriptors - Relay Descriptors |
| DEVICE PROG? 1 = YES 0 = NO 0 | For defining the following device characteristics for addressable devices, including keypads, RF receivers (5881/5882EU/5882EUH), relay modules (4204/4204CF), FSA modules, 4286 VIP Module, Panel Link Module and VISTA Gateway Module: - Device Address - Device Type - Keypad Options (including Partition assignment) - RF House ID - Panel Linking Options |
| OUTPUT PGM? 1 = YES 0 = NO 0 | For defining output relay functions. |
| RLY VOICE DESCR? 1 = YES 0 = NO 0 | For entering voice descriptors for relays to be used with the 4286/4286 VIP Module. |
| CUSTOM INDEX ? 1 = YES 0 = NO 0 | For creating custom word substitutes for VIP Module annunciation. |
| PROMPT | EXPLANATION |
| ACCESS POINT PGM 1 = YES 0 = NO 0 | For defining the parameters for each of the VistaKey access points, including which group(s) have access through an access point (door). See the VistaKey-SK Installation and Setup Guide for the detailed programming instructions. |
| ACCESS GRP PGM 1 = YES 0 = NO 0 | For defining the capabilities (privileges) for each group of users. See the VistaKey-SK Installation and Setup Guide for the detailed programming instructions. |
| EVENT/ACTION PGM 1 = YES 0 = NO 0 | For defining events and time periods for an access group. See the VistaKey-SK Installation and Setup Guide for the detailed programming instructions. |

### Zone Programming

Enter Programming mode [Installer Code] + 8 0 0 0. Program field 200 before beginning. Also, before programming wireless zones, the RF receiver must be enabled in Device Programming Mode (procedure described later in the Device Programming section). Then press #93 to display "ZONE PROG?".

| PROMPT | EXPLANATION |  |  |
| --- | --- | --- | --- |
| ZONE PROG? 1 = YES 0 = NO 0 | Press 1 to enter ZONE PROGRAMMING mode. The following screens will appear. Press [] to display the next screen. Press [#] to display a previous screen. |  |  |
| SET TO CONFIRM? 1 = YES 0 = NO 0 | This prompt appears once upon entering Zone Programming Mode. If “Yes,” Confirmation prompts will be displayed after the device’s Serial and Loop numbers have been entered later. |  |  |
| ENTER ZONE NO. 000 = Quit 010 Zone 010 entered  | Enter the 3-digit zone number to be programmed, as follows: Protection Zones = 001–128 Relay Zones = 601–696 (use for relays on 4204CF modules only). ECP Device Supervisory Zones = 800–830 System Supervisory Zones = 970, 988, 990, 992 (duress), 997 Keypad Panic Zones = 995, 996, 999 NOTE: Zones 601 through 830, 970, 988, 990, and 997 must be supervised and programmed with response type 05 (Day/Night) to comply with the “EN50131” specification. Press [] to continue. |  |  |
| 010 ZT P RC In L 00 1 10 00 1 | This display appears, showing a summary of the zone's current programming. ZT = Zone Type, P = Partition, RC = Report Code, In = the input type of device, and L = the device's loop number to which the sensor is connected. Some devices can support more than one zone by means of individual loops (for example, 5801, 5804, 5804EU, 5804H, 5804AP, etc.). If the zone is not programmed, the display appears as shown here. If you are checking a zone's programming, and it is programmed satisfactorily, press [#] to back up one step and enter another zone number, if desired. Press [] to continue. |  |  |
| 010 ZONE TYPE PERIMETER 03 Zone number 010 and Zone Type 03 entry shown † These are special zone types used with 5800/5800EU/5800H/5800AP Series RF devices and V-Plex keyswitches connected to 4193SNs. | Each zone must be assigned a zone type, which defines the way in which the system responds to faults in that zone. Refer to the Zone Type Definitions section in the VISTA-120IT Installation and Setup Guide for detailed definitions of each zone type. Enter the zone type desired (or change it, if necessary). Available zone types are listed below. |  |  |
|  | 00 = Assign for Unused Zones | 10 = Interior Delay, Burglary |  |
|  | 01 = Entry/Exit #1, Burglary | 14 = Panel Link Supervision |  |
|  | 02 = Entry/Exit #2, Burglary | 19 = 24-Hour Trouble |  |
|  | 03 = Perimeter, Burglary | 20 = Arm–STAY† |  |
|  | 04 = Interior Follower, Burglary | 21 = Arm AWAY† |  |
|  | 05 = Trouble Day/Alarm Night | 22 = Disarm† |  |
|  | 06 = 24 Hr. Silent Alarm | 23 = No Alarm Response (e.g., relay activation) |  |
|  | 07 = 24 Hr. Audible Alarm | 27 = Access Point |  |
|  | 08 = 24 Hr. Auxiliary | 28 = MLB Supervision (if VGM installed) |  |
|  | 09 = Fire Without Verification | 29 = Momentary Exit (used with VistaKey module) |  |
|  | Press [] to continue. |  |  |
| PROMPT | EXPLANATION |  |  |
| 010 ACCESS POINT (00-31) 01 | If response type 27, or 29 was selected, this prompt will be displayed. Enter the access point to be controlled by the response type (00-31 for type 27; 01-15 for type 29). Note: If you are using the VistaKey module, the access point must match the address that was set in the module. Press [] to continue. |  |  |
| 010 ENTRY OR EXIT 0 | If response type 27, or 29 was selected, this prompt will be displayed. Enter whether the access point is an entry or exit point. 0 = entry; 1 = exit Press [] to continue. |  |  |
| 010 Panel ID# (01-08) 01 | If you selected response type 14, this prompt appears. Enter the panel ID#. This ID # must match the ID # programmed in Device Programming. Press [] to accept entry. |  |  |
| 010 Partition 1 | Enter the partition number (1–8) you are assigning this zone to. Press [] to continue. |  |  |
| 010 REPORT CODE 1st 03 2nd 12 3C | Enter the report code. The report code consists of 2 hexadecimal digits, each in turn consisting of 2 numerical digits. For example, for a report code of "3C," enter 03 for "3" and 12 for "C." (Refer to the System Communication section in the Installation and Setup Guide for more information about report codes and reporting formats.) Press [] to continue. |  |  |
| 010 INPUT TYPE RF Xmitter 3 Input types 4 & 5 are valid for certain 5800/5800EU/5800H, 5800AP Series transmitters only (e.g., 5801, 5802, 5802CP, 5802AP, 5802EU, 5802H, 5804EU, 5804, 5804AP, 5804H). | Enter the input device type as follows: |  |  |
|  | 00 = not used |  | 07 = DIP switch-type polling loop device |
|  | 01 = basic wired |  | 08 = right loop of DIP switch type device |
|  | 02 = RF motion (RM type) (Zone restorals automatic) |  | 09 = keypad input (code + #73) |
|  |  |  | 10 = PassPoint ACS input |
|  | 03 = supervised RF transmitter (RF type) |  | 11 = VistaKey Door Status Monitor (DM) |
|  | 04 = unsupervised RF transmitter (UR type) |  | 12 = VistaKey Request to Exit (RE) |
|  | 05 = RF button-type transmitter (BR type) |  | 13 = VistaKey General Purpose (GP) |
|  | 06 = serial number polling loop device (SL type) |  |  |
|  | Right loops refer to the use of the right loop on a 4190SN/4193SN Zone Expander Module and/or 4278EX-SN PIR, which allow wired devices to be monitored by the polling loop. If you are programming wired or DIP switch polling loop devices, the summary display appears after completing this entry. NOTES: Input type 10 is applicable only if PassPoint ACS is installed with the VISTA Gateway Module. Input types 11 (DM), 12 (RE), and 13 (GP) should only be used with VistaKey modules. If you selected response type 14, 28, or 29 the input type MUST be 00. Press [] to continue. |  |  |
| 010 SMART CONTACT 1 = YES 0 = NO | If input type 3 or 6 was selected, this prompt will be displayed. Enter 1 for devices that monitor maintenance signals (ex. 5192SD, 5192SDT, 5808, Quest 2260SN) or can be used to limit fault signals in the disarmed state or have other smart capabilities (ex. Quest 2260SN). Otherwise, enter 0. NOTE: The Smart Contact option must ONLY be selected for devices that support smart feature(s), otherwise unpredictable results may occur. |  |  |
| 010 V-PLEX RELAY? 1 = YES 0 = NO | If you selected input type 6, this prompt displays. Enter 1 if using a 4101SN Relay Module for this zone. Otherwise enter 0. Press [] to continue. |  |  |
| 010 CONS ECP ADDR (00-30) 01 | If you selected input type 09, this prompt displays. Enter the ECP address of the keypad that is being used for entry/exit for this access point (00-30). Press [] to continue. |  |  |
| 010 ACS ZONE # (00-31) 01 | If you selected input type 10, this prompt displays. Enter the PassPoint ACS zone ID that this VISTA zone maps to (00-31). Press [] to continue. |  |  |
| PROMPT | EXPLANATION |  |  |
| 010 ACCESS POINT (01-15) 01 | If you selected input types 06, or 11 – 13, this prompt displays. Enter the access point (01-15) to be controlled by the input type. Note: For input type 06, the selected address must be 00. Note: If you are using the VistaKey module, the access point must match the address that was set in the module. Press [] to continue. |  |  |
| 010 INPUT S/N: L AXXX-XXXX1 | For Serial Number entry and Loop Number entry, do one of the following: a. Transmit two open and close (or close and open) sequences. For a button-type transmitter, press and release the button, wait approximately 4 seconds, then press and release the button a second time. OR b. Manually enter the 7-digit serial number printed on a label on the transmitter, using the Alpha keypad. Then press the [] key, the cursor moves to the “L” position. You can edit the loop number, if necessary. When the loop number is acceptable, press []. OR c. Press key [C] to copy the last serial number enrolled (used when programming a transmitter with several input loops). Press [] to accept. |  |  |
| 010 INPUT S/N: L A022-4064 1 | The cursor will then move to the Loop column (L) with the previously entered/transmitted serial number displayed. Enter the loop number (refer to 5800 Series Transmitters Loop Designations below). To Delete an Existing Serial Number, enter “0“ in the loop number field. The serial number will change to “0”s. If “0” was entered in error, simply re-enter the loop number or press [#], and the serial number will return to the display. Press [] to accept. |  |  |
| 010 INPUT S/N: L A022-4064 1 | The system will then check for a duplicate serial/loop number combination. If a duplicate serial/loop number combination is found, the keypad will emit a single long beep, and display the serial number along with a “?” for the loop number, allowing you to re-enter the correct loop number. If the serial/loop number combination is not a duplicate in the system, a display appears showing the serial number and loop number entry. Press [] to continue. |  |  |
| XMIT TO CONFIRM PRESS TO SKIP | Confirmation Option: This prompt only appears if you answered “Yes” at the first prompt. The system enters a confirmation mode so that the operation of the actual programmed input can be confirmed. Activate the loop input or button that corresponds to this zone. At any time during this step, you may press the [] key on the keypad to save the serial and loop number combination without confirming. |  |  |
| Entd A022-4063 1 Rcvd A022-4064 | If the serial number transmitted does not match the serial number entered, a display similar to the one at the left appears. If the loop number does not match, it is also displayed. If so, activate the loop input or button on the transmitter once again. If a match is not obtained (i.e., summary display does not appear), press the [#] key twice and then enter or transmit the correct serial number. |  |  |
| 010 ZT P RC In L 03 1 3C RF 1s | If the serial number transmitted does match the serial number entered, the system beeps 3 times and a summary display appears, showing that zone’s programming. Note that an “s” indicates that a transmitter’s serial number has been enrolled. Press [] to accept the zone information. |  |  |
| ENTER ZONE NO. 000 = Quit 011 | The system now returns to the “ENTER ZONE NO.” prompt for the next zone. When all zones have been programmed, enter “000” to quit. |  |  |

After you have enrolled each wireless device, remove ONE of the serial number labels from that device and affix it in the appropriate column on the worksheets provided later in this Programming Guide; then enter the other information (zone number, zone type, etc.) relevant to that device.

|  | When you have finished programming all zones, test each using the system's Test Mode. Do not use the Transmitter ID Sniffer Mode. The system checks only for transmission of one zone on a particular transmitter, NOT the zones assigned to each additional loop, and also does not verify polling loop type zones. |
| --- | --- |

### Expert Mode Zone Programming

Expert mode allows you to program zones using the minimum number of screens and keystrokes.

- Enter the Programming mode with [Installer Code] + 8 0 0 0

Before programming your zones, do the following:

- Program field 200: Number of Partitions.

- Enable your RF Receiver in Device Programming menu mode.

To program your zones, press 93 to display the "ZONE PROG?" prompt.  Enter “0” (NO) to each prompt until the “EXPERT MODE?” prompt appears.

NOTE: Some of the zone attributes cannot be programmed in the Expert Mode, only in Zone Programming.

| PROMPT | EXPLANATION |
| --- | --- |
| EXPERT MODE? 1 = YES 0 = NO 0 | Press 1 to enter Expert mode. |
| SET TO CONFIRM? 0 = NO 1 = YES 0 | This prompt appears once upon entering Expert Mode. If you select “Yes,” Confirmation prompts will be displayed after the device’s Serial and Loop numbers have been entered later. |
| Zn ZT P RC In L 001 03 1 10 HW - | A summary display appears, showing zone 1’s current programming or default values. |
| Zn ZT P RC In L 010 03 1 10 RF 1s | Enter the desired 3-digit zone number and press []. Note: If you want to exit the Expert mode, enter “000” + []. If an “s” appears after the loop number, it indicates that the transmitter’s serial number has been enrolled. Use the [D] key to enter and duplicate wireless keys (see “Entering Wireless Keys” later) |
| Zn ZT P RC In L 010 03 1 10 RF - | Enter all zone information except for Loop number, or press “C” to copy the zone information on this screen from the last saved zone (including Loop). ZT = Zone Type P = Partition RC = Report Code In = Input Device Type L = Loop number to which the sensor is connected. NOTE: Pressing the [C] copies the zone information from the last saved zone, which includes the input type. Verify this information is correct for this zone. On this screen: - Use the [A] key to move to the right. - Use the [B] key to move to left and to back up to “ZT” field. Press [] to accept the existing or newly-entered zone information. |
| ZN B M V A C E AD 010   1 0 1 1 01 | Enter the remainder of the zone’s information, or press the [C] key to copy the zone attributes on this screen from the last saved zone. B = Not Applicable M = Not Applicable V = V-plex Relay? (only used if “In” = 6) AC = Access Point (only used if ZT = 27, 29 or In = 6, 11, 12, 13) E = Entry or Exit? (only used if ZT = 27) AD = Address (only used if “In” = 9 or 10) If “In” = 9, enter the Device Address If “In” = 10, enter the PassPoint Zone Number NOTE: Pressing the [C] copies the zone attributes from the last saved zone. Verify the attributes for this zone are correct. On this screen: - Use the [A] key to move to the right. - Use the [B] key to move to left and to back up to “V” field. Press [] to accept existing information. |
| PROMPT | EXPLANATION |
| 010 INPUT S/N: L A XXX-XXXX - | If you entered RM, RF, BR, UR or SL for the Input Type, this screen displays. Otherwise the summary screen for the next zone displays. Enter the 7-digit serial number, using one of the following methods: a. Transmit two open and close (or close and open) sequences. For a button-type transmitter, press and release the button, wait approximately 4 seconds, then press and release the button a second time. OR b. Manually enter the 7-digit serial number printed on a label on the transmitter, using the alpha keypad. Then press the [] key, the cursor will move to the “L” position. You can edit the loop number, if necessary. When the loop number is acceptable, press []. OR c. Press key [C] to copy the last serial number enrolled (used when programming a transmitter with several input loops). Remember, you can use the [A] key to move to the right or the [B] key to move to the left. You can also use the [#] key to back up without saving. |
| 010 INPUT S/N: L A022-4064 1 | Press [] to accept the serial number and advance to the “L” position (if method “a” or “c” was used), then enter the loop number. If necessary, press the [#] key to back up without saving and re-enter or edit the serial number before pressing [] to save The system checks for a duplicate. If a duplicate serial/loop number combination is found, the keypad will emit a single long beep, and display the serial number along with a “?” for the loop number, allowing you to re-enter the correct loop number. |
| 010 INPUT S/N: L A000-0000 1 | To Delete an Existing Serial Number, enter “0“ in the loop number field. The serial number will change to “0”s. If “0” was entered in error, simply re-enter the loop number or press [#], and the serial number will return to the display. |
| XMIT TO CONFIRM PRESS TO SKIP | The prompt to confirm appears. This prompt only appears if you answered “Yes” at the “SET TO CONFIRM?” prompt. The system enters a confirmation mode so that the operation of the actual programmed input can be confirmed. Activate the loop input or button that corresponds to this zone. At any time during this step, you may press the [] key on the keypad to save the serial and loop number combination without confirming. |
| Entd A022-4063 1 Rcvd A022-4064 | If the serial number transmitted does not match the serial number entered, a display similar to the one at the left appears. If the loop number does not match, it also is displayed. If so, activate the loop input or button on the transmitter once again. If a match is not obtained (i.e., summary display for the next zone does not appear), press the [#] key twice and then enter or transmit the correct serial number. Activate the button on the wireless key again after re-entering the serial number. |
| Zn ZT P RC In L 011 00 1 10 00 1 | If the serial number transmitted matches the serial number entered, the system beeps 3 times and advances to the summary display for the next zone's programming. After all the zones have been programmed, enter 000 for the zone number to quit. |

After you have enrolled each wireless device, remove ONE of the serial number labels from that device and affix it in the appropriate column on the worksheets provided later in this Programming Guide; then enter the other information (zone number, zone type, etc.) relevant to that device.

#### Entering Wireless Keys

If you pressed the D key previously to enter defaults for 5804 and/or 5804BD wireless keys, the following screens appear:

| PROMPT | EXPLANATION |
| --- | --- |
| FROM TEMPLATE 1–6 1 | Enter template number (1–6). 1–3 = 5804/5804AP/5804EU/5804H templates; 4–6 = 5804BD templates. See the defaults provided for each template in the chart that follows these procedures. Select from templates. Press [] to display template (template 1 shown selected). Note: If necessary, press [#] to back up and re-enter template number. Press [#] if you want to return to zone attributes screen. |
| L 01 02 03 04 ZT 23 22 21 23 1 | When you press [], the selected template is displayed. Top line of display represents loop numbers; bottom line represents zone type. Press [] to accept template. |
| PARTITION 1 | Enter partition number for wireless key. Press [] to continue. |
| ENTER ZONE NO 000 = QUIT 024  Example of zone number suggested by the system. This indicates that zones 24, 25, 26, and 27 are available. | The system searches for the highest available, consecutive 4-zone group (the four zones required for the 5804, 5804AP, 5804EU and 5804H), and displays the lowest zone number of the group. If you want to start at a different zone number, enter the zone desired and press []. If that zone number is displayed, the system has the required number of consecutive zones available, beginning with the zone you entered. If not, the system again displays a suggested zone that can be used. If the required number of consecutive zones is not available at all, the system will display “000.” Press [] to accept. |
| 024 INPUT S/N L AXXX-XXXX 1 | To enter the serial number: Press and release a button on the wireless key. OR Manually enter the 7-digit serial number printed on the device’s label. Press [] to accept serial number. The system checks for a duplicate. If a duplicate exists, a long error beep will sound and the serial number reverts back to all “X”s allowing you to re-enter the serial number. Use the [A] key to move forward within the screen, and the [B] key to back up. |
| XMIT TO CONFIRM PRESS TO SKIP | If you entered YES previously at the SET TO CONFIRM prompt (see first prompt following entry into the Expert Programming Mode), the display on the left appears. To confirm, activate the button on the wireless key that corresponds to this zone. |
| Entd A022-4063 Rcvd A022-4064 | If the serial number transmitted does not match the serial number entered, a display similar to the one at the left appears. If so, activate the loop input or button on the transmitter once again. If a match is not obtained (i.e., summary display does not appear), press the [#] key and then enter the correct serial number. Activate the button on the wireless key again after re-entering the serial number. |
| ENTER ZONE NO 000 = QUIT 028 | If the serial number transmitted matches the serial number entered, the system will beep 3 times and revert to the "Start Zone No." prompt and will show the lowest numbered zone of the next available 4-zone group (4 consecutive zones) that is available for programming. After all the wireless keys have been entered, enter 000 for the zone number to quit. |

After you have enrolled each wireless device, remove ONE of the serial number labels from that device and affix it in the appropriate column on the worksheets provided later in this Programming Guide; then enter the other information (zone number, zone type, etc.) relevant to that device.

##### Wireless Key Default Templates

| 5804/5804AP/ 5804EU/5804H |  |  |  | 5804BD |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Template 1 | Loop | Function | Zone Type | Template 4 | Loop | Function | Zone Type |
|  | 1 | No Response | 23 |  | 1 | No Response | 23 |
|  | 2 | Disarming | 22 |  | 2 | No Response | 23 |
|  | 3 | Arm AWAY | 21 |  | 3 | Arm AWAY | 21 |
|  | 4 | No Response | 23 |  | 4 | Disarming | 22 |
| Template 2 | Loop | Function | Zone Type | Template 5 | Loop | Function | Zone Type |
|  | 1 | No Response | 23 |  | 1 | No Response | 23 |
|  | 2 | Disarming | 22 |  | 2 | Arm STAY | 20 |
|  | 3 | Arm AWAY | 21 |  | 3 | Arm AWAY | 21 |
|  | 4 | Arm STAY | 20 |  | 4 | Disarming | 22 |
| Template 3 | Loop | Function | Zone Type | Template 6 | Loop | Function | Zone Type |
|  | 1 | 24-Hour Panic | 07 |  | 1 | 24-Hour Panic | 07 |
|  | 2 | Disarming | 22 |  | 2 | Arm STAY | 20 |
|  | 3 | Arm AWAY | 21 |  | 3 | Arm AWAY | 21 |
|  | 4 | Arm STAY | 20 |  | 4 | Disarming | 22 |

### Report Code Programming

All report codes are entered using #93 menu mode programming, either through Report Code Programming, or through the Zone Programming mode while entering other zone information.  In the VISTA-120IT, reports are divided into six categories. These categories represent the main menu options found in the Report Code Programming mode.  Reports and the categories in which they are found are as follows:

| ALARM CODES | RESTR, SUPV. CODES (for groups of 16 zones) | SYSTEM GROUP #1 | SYSTEM GROUP #2 | SYSTEM GROUP #3 | SYSTEM GROUP #4 |
| --- | --- | --- | --- | --- | --- |
| Zone Alarm Reports | Alarm Restore Trouble Trouble Restore Bypass Bypass Restore | Closing (arm AWAY) Opening (disarm) System Low Battery Low Battery Restore AC Mains Loss AC Mains Restore Periodic Burglary Test Power On Reset Cancel Program Tamper | Arm STAY Time Set, Log Reset Log 50%, 90% Full Event Log Overflow Exit Error Zone Exit Error by User Recent Close | Early Open Early Close Late Open Late Close Failed to Open Failed to Close Auto-Arm Failed Schedule Change | Walk Test Start Walk Test End |

The programming sequence that follows assumes that you will be entering all reports for the system at one time. In actuality, you may skip from one main menu option to another by pressing 0 (N) to each main menu option.

To enter report codes, do the following:

Enter Program mode [Installer Code] + 8 0 0 0, then press #93.  Enter (N) to the main menu options until the Report Code Programming option is displayed.

| PROMPT | EXPLANATION |
| --- | --- |
| REPORT CODE PROG 1 = YES 0 = NO 0 | Press 1 (Y) to enter to Report Code Programming. |

#### Zone Alarm Reports

| PROMPT | EXPLANATION |
| --- | --- |
| ALARM, ID DIGIT? 1 = YES 0 = NO 0 | Press [1] (Y) to enter Alarm Report Codes for zones. Press [0] (N) to skip to the next main menu option. |
| PROMPT | EXPLANATION |
| ENTER ZONE NO. 000 = QUIT 001 | Enter the zone number for which you are entering the report code. Press [] to continue. |
| 001 REPORT CODE 1st 00 2nd 00 00 | Enter the first digit of the Alarm report code (double-digit entry) and press []. Enter the 2nd digit of the Alarm Report code. Press [] to continue. |
| ENTER ZONE NO. 000 = QUIT 001 | Enter the zone number for which you are entering the report code. When all zone Alarm Codes have been programmed, enter 000 to Quit. Press [] to continue. |
| QUIT REPORT MENU 1 = YES 0 = NO 0 | If you have completely finished entering report codes, press [1] (Y) to quit Report Code Programming. If you wish to enter other system report codes, enter 0 (N). Press [] to continue. |

#### Restore/Supervisory Codes

| PROMPT | EXPLANATION |
| --- | --- |
| RESTR, SUPV. CODE 1 = YES 0 = NO 0 | Press [1] (Y) to enter Restore and Supervisory Codes for zones. |
| ENTER ZN FOR GRP 000 = QUIT 001 | Enter one zone for each group of 16 zones (001-016, 017-032, etc.). |
| ALARM RESTORE GRP 001-016 00 | Enter the first digit of the Alarm Restore Report Code for this group of zones (double-digit entry). The second digit (for two-digit reporting formats) is automatically the ID (second) digit of the Alarm Report Code for each zone (if programmed). Press [] to continue. |
| TROUBLE GRP 001-016 00 | Enter the first digit of the Trouble Report Code for this group of zones (double-digit entry). The second digit (for two-digit reporting formats) is automatically the ID (second) digit of the Alarm Report Code for each zone (if programmed). Press [] to continue. |
| TROUBLE RESTORE GRP 001-016 00 | Enter the first digit of the trouble restore code (single-digit entry) and press []. The second digit (for two-digit reporting formats) is automatically the ID (second) digit of the alarm report code for each zone (if programmed). Press [] to continue. |
| BYPASS GRP 001-016 00 | Enter the first digit of the Bypass Report Code (double-digit entry) and press []. The second digit (for two-digit reporting formats) is automatically the ID (second) digit of the Alarm Report Code for each zone (if programmed). Press [] to continue. |
| BYPASS RESTORE GRP 001-016 00 | Enter the first digit of the Bypass Restore Report Code (double-digit entry) and press []. The second digit (for two-digit reporting formats) is automatically the ID (second) digit of the Alarm Report Code for each zone (if programmed). Press [] to continue. |
| ENTER ZN FOR GRP 000 = QUIT 017 | Enter one zone for each group of 16 zones. When you are finished entering Restore and Supervisory Codes for all zone groups, enter 000. Press [] to continue. |
| QUIT REPORT MENU 1 = YES 0 = NO 0 | If you have completely finished entering report codes, press [1] (Y) to quit Report Code Programming. If you wish to enter other system report codes, enter 0 (N). Press [] to continue. |

#### System Group #1 Codes

| PROMPT | EXPLANATION |
| --- | --- |
| SYSTEM GROUP #1? 1 = YES 0 = NO 0 | To enter System Group #1 codes, press 1 (Y). |
| CLOSE 1st 00 2nd 00 | Enter the first digit of the Closing (Arm-AWAY) report. Press []. Enter the second digit of the report. If the user number is desired as the second digit, enter 01 (not necessary for Contact ID or High Speed formats). Press [] to continue. |
|  | Enter the rest of the codes in the same manner. Other report codes in System Group #1 are: - Opening (Disarm) - System Low Battery - Low Battery Restore - AC Mains Loss - AC Mains Restore - Periodic Test - Power On Reset - Cancel - Program Tamper Once you have entered these report codes, the system prompts you with the Quit menu. |
| QUIT REPORT MENU 1 = YES 0 = NO 0 | If you have completely finished entering report codes, press 1 (Y) to quit Report Code Programming. If you wish to enter other system report codes, enter 0 (N). Press [] to continue. |

#### System Group #2 Codes

| PROMPT | EXPLANATION |
| --- | --- |
| SYSTEM GROUP #2 ? 1 = YES 0 = NO 0 | To enter System Group #2 codes, press [1] (Y). |
| STAY 1st 00 2nd 00 | Enter the first digit of the Arm-STAY report. Press []. Enter the second digit of the report. If the user number is desired as the second digit, enter 01 (not necessary for Contact ID or High Speed formats). Press [] to continue. |
|  | Enter the rest of the codes in the same manner. Other codes in System Group #2 are: - Time Set, Log Reset - Log 50%, 90% Full - Event Log Overflow - Exit Error Zone - Exit Error by User - Recent Close Once you have entered these report codes, the system prompts you with the Quit menu. |
| QUIT REPORT MENU 1 = YES 0 = NO 0 | If you have completely finished entering report codes, press [1] (Y) to quit Report Code Programming. If you wish to enter other system report codes, enter 0 (N). Press [] to continue. |

#### System Group #3 Codes

| PROMPT | EXPLANATION |
| --- | --- |
| SYSTEM GROUP #3 ? 1 = YES 0 = NO 0 | To enter System Group #3 codes, press [1] (Y). |
| PROMPT | EXPLANATION |
| EARLY OPEN 1st 00 2nd 00 | Enter the first digit of the Early Opening Report Code. Press []. Enter the second digit of the report code. If the user number is desired as the second digit, enter 01 (not necessary for Contact ID or High Speed formats). Press [] to continue. |
|  | Enter the rest of the codes in the same manner. Other codes in System Group #3 are: - Early Close - Late Open - Late Close - Failed to Open - Failed to Close - Auto-Arm Failed - Schedule Change Once you have entered these report codes, the system prompts you with the Quit menu. |
| QUIT REPORT MENU 1 = YES 0 = NO 0 | If you have completely finished entering report codes, press [1] (Y) to quit Report Code Programming. If you wish to enter other system report codes, enter 0 (N) Press [] to continue. |

#### System Group #4 Codes

| PROMPT | EXPLANATION |
| --- | --- |
| SYSTEM GROUP #4 ? 1 = YES 0 = NO 0 | To enter System Group #4 codes, press [1] (YES). |
| WALK TEST START 1st 00 2nd 00 | Enter the first digit of the Walk Test Start Report Code. Press []. Enter the second digit of the report code. Press [] to continue. |
|  | Enter the rest of the codes in the same manner. Other codes in System Group #4 are: - Walk-Test End. Once you have entered these report codes, the system prompts you with the Quit menu. |
| QUIT MENU MODE? 1 = YES 0 = NO 0 | Enter 1 to exit back to normal Programming mode. Enter 0 to stay in Menu mode. |

### Alpha Descriptor Programming

You can program a user friendly English language description/location keypad displays for all protection zones, relays, keypad panics, polling loop short, and RF receiver supervision troubles.  Each description can be composed of a combination of words (up to a maximum of 3) selected from a vocabulary of 244 words stored in memory, and any word can have an "s" or " 's " added to it.

Also, up to 60 installer-defined words can be added to the memory.  Thus, when an alarm or trouble occurs in a zone, an appropriate description for the location of that zone is displayed at the keypad.  A custom installer's message can be programmed for each partition, which will be displayed when the system is "Ready" (ex. THE PETERSON'S).

- To program alpha descriptors, enter Programming mode, then press #93 to display "ZONE PROG?"

- Press [0] (NO) twice to display "ALPHA PROG?".

- Press [1] to enter Alpha Programming.

There are 6 submenu selections that will be displayed one at a time.

Press [1] to select the mode desired.  The alpha menu selections are:

ZONE DESCRIP?	for entering zone descriptors.

DEFAULT SCREEN?	for creating custom message; displayed when system ready.

CUSTOM WORD?	for creating custom words for use in descriptors.

PART DESCRIP?	for creating 4-character partition names.

EXIT EDIT MODE?	to exit Alpha Programming

- Refer to the sections that follow for adding alpha descriptors.

#### Zone Descriptors

- Select ZONE DESCRIPTOR mode.

The keypad keys perform the following functions:

[3]	Scrolls both alphabet and actual words in ascending alphabetical order.

[1]	Scrolls both alphabet and actual words in descending alphabetical order.

[2]	Adds or removes an "s" or " 's " to a vocabulary word.

[6]	Switches between alphabet and actual word list; used to accept entries.

[8]	Saves the zone description in the system's memory.

[#]	[#] plus zone number displays the description for that zone.

- Enter the zone number to which you want to assign a descriptor.

For example, key [] 001 to begin entering the description for Zone 1, (key [] 002 for Zone 2, [] 003 for Zone 3, etc.). The following is displayed:  ZN 001 A.

Note that the first letter of the alphabet appears after the zone number, and that the zone number is automatically included with the description.

- Enter the descriptor for that zone.

Use one of two methods as follows:

(Assume, for example, that the desired description for Zone 1 is BACK DOOR.)

- Press [#] followed by the 3-digit number of the first word from the fixed dictionary shown later in this section (e.g., [0][1][3] for BACK).

Press [6] in order to accept the word and proceed, or press [8] to store the complete descriptor and exit;

or

b)	Select the first letter of the desired description (note that "A" is already displayed).  Use the [3] key to advance through the alphabet and the [1] key to go backward.

Press [3] key repeatedly until "B" appears (press [1] to go backwards if you happen to pass it), then press [6] to display the first available word beginning with “B”.

Press [3] repeatedly to advance through the available words until the word “BACK” is displayed.

|  | To add an "s" or " 's," if you need to, press 2. The first depression adds an "s," the second depression adds an " 's, " the third depression displays no character (to erase the character), the fourth depression adds an "s," etc. |
| --- | --- |

- Accept the word.

To accept the word, press [6], which switches back to the alphabet list for the next word, or press [8] to store the complete descriptor and then exit.

- Select the next word.

For selection of the next word (DOOR), repeat step 3a (word #057) or 3b, but selecting the word "DOOR.”

To accept the word, press [6], which again switches back to alphabet list.

- Store the descriptor.

When all desired words have been entered, press [8] to store the description in memory.

To review the zone descriptors, key [#] plus zone number (e.g., #001).

To edit zone descriptors, key [] plus zone number (e.g., 001)

- Exit Zone Description Mode: enter 000.

#### Default Screen (Custom Message Display)

Normally, when the system is in the disarmed state, the following display is present on the keypad.

****DISARMED****

- READY TO ARM

Part or all of the above message can be modified to create a custom installer message for each partition. For example, “****DISARMED****” on the first line or “READY TO ARM” on the second line could be replaced by the installation company name or phone number for service.

Note: There are only 16 character spaces on each of the two lines.

To create a custom display message, proceed as follows:

- Select Default Screen mode.

The keypad asks for the partition number for this message.

Enter the partition number. Press [] to accept entry.  The following display appears:

****DISARMED****

READY TO ARM

A cursor is present at the extreme left of the first line (over the first "star"). Press [6] to move the cursor to the right and [4] to move the cursor to the left. Press [7] to insert spaces or erase existing characters.

- Create the message.

For example, to replace “READY TO ARM” with the message “SERVICE 424-0177,” proceed as follows:

Press [6] to move the cursor to the right, and continue until the cursor is positioned over the first location on the second line.

Press [3] to advance through the alphabet to the first desired character (in this case, "S"). Press [1] to go backward, when necessary. When the desired character is reached, press [6].

The cursor then moves to the next position, ready for entry of the next character (in this example, "E"). When the cursor reaches a position over an existing character, press [3] or [1] to advance or back up from that character in the alphabet.

Proceed in this manner until all characters in the message have been entered.

- Save the message.

Store the new display message in memory by pressing [8].

- The system asks for a new partition number.

Enter 0 to quit or 1-8 for a new partition number.

#### Custom Words

Up to 60 installer-defined words can be added to the built-in vocabulary. Each of the 60 "words" can actually consist of several words, but bear in mind that a maximum of 10 characters can be used for each word string.

- Select CUSTOM WORD Mode.

The keys perform the following functions:

[3]	Advances through alphabet in ascending order.

[1]	Advances through alphabet in descending order.

[6]	Selects desired letter; moves the cursor 1 space to the right.

[4]	Moves the cursor one space to the left.

[7]	Inserts a space at the cursor location, erasing any character at that location.

[8]	Saves the new word in the system's memory.

[]	Returns to Description Entry Mode.

- Enter the custom word number (01-60) you want to create.

For example, if you are creating the first word (or word string), enter 01; when creating the second word, enter 02, and so on. A cursor now appears at the beginning of the second line.

- Type the word using one of two methods as follows:

a)	Press [#], followed by the 2-digit entry for the first letter you would like to display (e.g., 65 for "A").

When the desired character appears, press [6] to select it. The cursor will then move to the right, in position for the next character. Press [#] plus the 2-digit entry for the next letter of the word.

or

- Press 3 to advance through the list of symbols, numbers, and letters.

Press 1 to move back through the list.

When you have reached the desired character, press [6] to select it. The cursor then moves to the right, in position for the next character.

- Repeat step 3 to create the desired custom word (or words).

Press [4] to move the cursor to the left if necessary.

Press [7] to enter a blank (or to erase an existing character).

Each word or word string cannot exceed 10 characters.

- Save the word by pressing [8].

This returns you to the “CUSTOM WORD?” display.  The custom word (or string of words) is automatically added to the built-in vocabulary at the end of the group of words beginning with the same letter.

Custom words are retrieved as word numbers 245 to 304 for words 1 to 60, respectively, when using method 3a to enter alpha descriptors.

When using method 3b to enter alpha descriptors, each word appears at the end of the group of words that begin with the same letter as it does.

- Repeat steps 2 through 6 to create up to a maximum of 60 custom words (or word strings).

- Exit Custom Word Mode by entering 00 at the “CUSTOM WORD” prompt.

#### Partition Descriptors

- Select "Part DESCRIPT." Mode.

The system asks for the partition number desired. Enter the number as a single-key entry 1-8.

- Follow the same procedure as for custom words.

Note: The partition descriptors are limited to 4 characters (e.g., WHSE for warehouse).

### ALPHA DESCRIPTOR VOCABULARY

(For Entering Alpha Descriptors.  To select a word, press [#] followed by the word's 3-digit number.)

NOTE: This vocabulary is not to be used for relay voice descriptors.  See Relay Voice Descriptors later in this section when programming relay voice descriptors.

000	(Word Space)

•	001	AIR

•	002	ALARM

003	ALCOVE

004	ALLEY

005	AMBUSH

•	006	AREA

•	007	APARTMENT

008	ART

•	009	ATTIC

010	AUDIO

011	AUXILIARY

•	012	BABY

•	013	BACK

•	014	BAR

015	BARN

•	016	BASEMENT

•	017	BATHROOM

•	018	BED

•	019	BEDROOM

020	BELL

•	021	BLOWER

•	022	BOILER

023	BOTTOM

024	BOX

025	BREAK

•	026	BUILDING

027	BURNER

028	CABINET

•	029	CALL

030	CAMERA

031	CAR

032	CASE

033	CASH

034	CCTV

035	CEILING

036	CELLAR

•	037	CENTRAL

038	CIRCUIT

039	CLIP

•	040	CLOSED

041	COIN

042	COLD

043	COATROOM

044	COLLECTION

045	COMBUSTION

•	046	COMPUTER

047	CONTACT

•	048	DAUGHTERS

049	DELAYED

•	050	DEN

051	DESK

•	052	DETECTOR

•	053	DINING

054	DISCRIMINATOR

055	DISPLAY

056	DOCK

•	057	DOOR

058	DORMER

•	059	DOWN

•	060	DOWNSTAIRS

061	DRAWER

•	062	DRIVEWAY

063	DRUG

•	064	DUCT

•	065	EAST

066	ELECTRIC

067	EMERGENCY

068	ENTRY

•	069	EQUIPMENT

070	EXECUTIVE

•	071	EXIT

072	EXTERIOR

•	073	FACTORY

074	FAILURE

075	FAMILY

•	076	FATHERS

•	077	FENCE

078	FILE

•	079	FIRE

•	080	FLOOR

081	FLOW

082	FOIL

•	083	FOYER

084	FREEZER

•	085	FRONT

086	FUR

087	FURNACE

088	GALLERY

•	089	GARAGE

•	090	GAS

091	GATE

•	092	GLASS

093	GUEST

094	GUN

•	095	HALL

•	096	HEAT

097	HIGH

098	HOLDUP

099	HOUSE

100	INFRARED

•	101	INSIDE

102	INTERIOR

103	INTRUSION

104	JEWELRY

•	105	KITCHEN

•	106	LAUNDRY

•	107	LEFT

108	LEVEL

•	109	LIBRARY

•	110	LIGHT

111	LINE

112	LIQUOR

•	113	LIVING

•	114	LOADING

115	LOCK

116	LOOP

117	LOW

•	118	LOWER

•	119	MACHINE

120	MAGNETIC

121	MAIDS

122	MAIN

•	123	MASTER

124	MAT

•	125	MEDICAL

126	MEDICINE

127	MICROWAVE

128	MONEY

129	MONITOR

•	130	MOTHERS

•	131	MOTION

132	MOTOR

133	MUD

•	134	NORTH

135	NURSERY

•	136	OFFICE

137	OIL

•	138	OPEN

139	OPENING

•	140	OUTSIDE

141	OVERFLOW

142	OVERHEAD

143	PAINTING

•	144	PANIC

145	PASSIVE

•	146	PATIO

147	PERIMETER

•	148	PHONE

149	PHOTO

150	POINT

151	POLICE

152	POOL

•	153	POWER

154	QUAD

155	RADIO

•	156	REAR

157	RECREATION

158	REFRIG

159	REFRIGERATION

160	RF

•	161	RIGHT

•	162	ROOM

163	ROOF

164	SAFE

165	SCREEN

166	SENSOR

•	167	SERVICE

•	168	SHED

169	SHOCK

•	170	SHOP

171	SHORT

172	SHOW

•	173	SIDE

174	SKYLIGHT

175	SLIDING

•	176	SMOKE

177	SONIC

•	178	SONS

•	179	SOUTH

180	SPRINKLER

181	STAMP

•	182	STATION

183	STEREO

184	STORE

•	185	STORAGE

186	STORY

187	STRESS

188	STRIKE

189	SUMP

190	SUPERVISED

191	SUPERVISION

192	SWIMMING

193	SWITCH

194	TAMPER

195	TAPE

196	TELCO

197	TELEPHONE

198	TELLER

•	199	TEMPERATURE

200	THERMOSTAT

•	201	TOOL

202	TRANSMITTER

203	TRAP

204	ULTRA

•	205	UP

•	206	UPPER

•	207	UPSTAIRS

•	208	UTILITY

209	VALVE

210	VAULT

211	VIBRATION

212	VOLTAGE

213	WALL

214	WAREHOUSE

215	WASH

•	216	WEST

•	217	WINDOW

218	WINE

•	219	WING

220	WIRELESS

221	WORK

222	XMITTER

223	YARD

224	ZONE  (No.)

•	225	ZONE

226	0

227	1

228	1ST

229	2

230	2ND

231	3

232	3RD

233	4

234	4TH

235	5

236	5TH

237	6

238	6TH

239	7

240	7TH

241	8

242	8TH

243	9

244	9TH

245	Custom Word 1

to

304	Custom Word 60

CHARACTER (ASCII) CHART (For Adding Custom Words)

32	(space)

33	!

34	"

35	#

36	$

37	%

38	&

39	'

40	(

41	)

42	*

43	+

44	,

45	-

46	.

47	/

48	0

49	1

50	2

51	3

52	4

53	5

54	6

55	7

56	8

57	9

58	:

59	;

60	<

61	=

62	>

63	?

64	@

65	A

66	B

67	C

68	D

69	E

70	F

71	G

72	H

73	I

74	J

75	K

76	L

77	M

78	N

79	O

80	P

81	Q

82	R

83	S

84	T

85	U

86	V

87	W

88	X

89	Y

90	Z

Notes: This factory-provided vocabulary of words is subject to change.

Bulleted words in bold face type are those that are also available for use by the 4286 VIP module.  If using a VIP module, and words other than these are selected for alpha descriptors, the voice module will not provide annunciation of those words.

### Device Programming

This menu is used to program keypads, receivers and relay modules.

From Data Field Programming mode, press #93 to display "ZONE PROG?". Press 0 repeatedly to display "DEVICE PROG?".

| PROMPT | EXPLANATION |  |
| --- | --- | --- |
| DEVICE PROG? 1=YES 0=NO 0 | Press [1] to enter Device Programming. |  |
| DEVICE ADDRESS 01-30, 00=QUIT 01 | The device address identifies the device to the control. Enter the 2-digit device address number to match the device's physical address setting (01-30). Note: Device Address 04 must be used for the 4286 Voice Module, if one is utilized. If not, it can be used for another device type. Press [] to accept entry. |  |
| DEVICE TYPE 00 | Select the type of addressable device as follows: |  |
|  | 00 = device not used | 06 = long range radio or TCP-IP network interface |
|  | 01 = alpha keypad (6139/6160/6164, AUI) | 08 = fire display (FSA) |
|  | 03 = RF receiver (5881/5882) | 09 = Vista Gateway Module (VGM) |
|  | 04 = output relay module (4204) | 10 = Panel Link Module |
|  | 05 = voice (VIP) Module (4286) |  |
|  | Press [] to accept entry. |  |

#### Alpha Keypad

| PROMPT | EXPLANATION |
| --- | --- |
| 01 CONSOLE PART. 1 | If you selected device type 01 (alpha keypad), this prompt appears. Enter the addressable device's default partition number (01 to maximum number of partitions programmed for system in field 2*00). This is the primary partition for the device. Enter 9 to make this keypad a "Master" keypad for the system. Press [] to accept entry. |
| 01 SOUND OPTION 00 | If you entered device type 01 (alpha keypad), this prompt appears. Keypads can be individually programmed to suppress arm/disarm beeps, entry/exit beeps and chime mode beeps. This helps prevent unwanted sounds from disturbing users in other areas of the premises. Enter a number 00-03 for the keypad sounding suppression options desired for the keypad as follows: 00 = no suppression 01 = suppress arm/disarm & entry/exit beeps 02 = suppress chime mode beeps only 03 = suppress arm/disarm, entry/exit and chime mode beeps Press [] to accept entry. |
| 01 KEYPAD GLBL? 0 | If you entered device type 01 (alpha keypad), this prompt appears. Each keypad can give users with access to multiple partitions the ability to arm and disarm those partitions from it. To enable this keypad for global arming/disarming, enter 1. To prevent the ability to use this keypad for global arming/disarming, enter 0. Press [] to accept entry. |
| 01 AUI ? 1 = YES 0 = NO 0 | If you selected device type 01 (alpha keypad), this prompt appears. Enter 1 (YES) if the device is an AUI (Symphony 8112, 8122, 8132, 8142, or 6270). See the Symphony’s instructions for the installation and operation. Press [] to accept entry. |

#### RF Expander

| PROMPT | EXPLANATION |
| --- | --- |
| 01 RF EXPANDER HOUSE I 00 | If you selected device type 03 (RF receiver), this prompt appears. Enter the 2-digit House ID (00-31). This is required if you are using a wireless keypad (5827/5827AP). Press [] to accept entry. |

#### Output Relay Module

| PROMPT | EXPLANATION |
| --- | --- |
| 01 SUPERVISED CF? 0 | If you selected device type 04 (relay module), this prompt appears. Enter 1 if the unit is a 4204CF. If not, enter 0. If you enter 1, only Relay 1 (Output A) and Relay 3 (Output B) on each module may be programmed for functions. This module is usually used to add two additional supervised bell outputs to the system. Press [] to accept entry. |

#### Panel Linking Module

| PROMPT | EXPLANATION |
| --- | --- |
| 01 PANEL ID# (01-08) 01 | If you selected device type 10 (Panel Linking Module), this prompt appears. Enter the panel’s ID # (01-08). Each panel must have its own unique ID #. This device is supervised as zone “8xx,” where “xx” = the device’s address. If you want to supervise this device, program that zone with response type 05. If you want to supervise Panel Link Modules from other controls, also program them into Zone Programming on a regular zone (001-128) with a response type of 14. Press [] to accept entry. |

#### VIP Module and VISTA Gateway Module

| PROMPT |  | EXPLANATION |
| --- | --- | --- |
| 01 MODULE PART. 1 |  | If you selected device type 05 (4286) or type 09 (VGM), this prompt appears. Enter the partition number 1-8 in which the module is located. Press [] to accept entry. Press 00 + [] to exit Menu mode. Press 99 to exit Program mode. |
|  | Device Address 00 is always set as an alpha keypad assigned to Partition 1 with no sounder suppression options. |  |

### Output Programming

The system supports up to 96 outputs.  Outputs can be used to perform many different functions and actions.  Each output must be programmed to begin one of four types of ACTIONS at a designated START event, and end that ACTION at a designated STOP event.  The options used to start and stop these devices are described below, followed by the actual screen prompts and available entries.

Note: No more than 10 relays can be programmed to respond to the same function/action.

The letter(s) in parentheses after each function described below, such as (A) after ACTION, are those that appear in the various summary displays of programmed data during programming.

ACTION (A)	The "ACTION" of the device is how the device will respond when it is activated by the "START" programming.  You may want the device to activate momentarily, to pulse on and off continuously, or to remain activated until some other event occurs to stop it.  There are five different action choices:

- ACTIVATE for 2 SECONDS and then reset.

- ACTIVATE and remain activated until stopped by some other event.

- PULSE ON and OFF until stopped by some other event (do not use with an FSA device).

- NO RESPONSE when the device is not used.

- TOGGLE on and off alternately with each activation of event.

START (STT)	The "START" programming determines when and under what conditions the device is activated.  The following START options are available:

1)	EVENT (EV) is the condition (alarm, fault, trouble) that must occur to a zone or group of zones (zone list) in order to activate the device. These conditions apply only when a zone list is used.  The different choices for "EVENT" are listed below and in “Programming Relays” later in this section.

- ALARM	Action begins upon any alarm in an assigned zone in the zone list.

- FAULT	Action begins upon any opening of an assigned zone in the zone list.

- TROUBLE	Action begins upon any trouble condition in an assigned zone in the zone list.

- NOT USED	Action is not dependent upon one of the above events.

ZONE LIST (ZL) is a group of zones to which the “EVENT” applies in order to activate a particular device.  Note that there are a total of 15 zone lists that can be programmed; when the selected EVENT (alarm, fault or trouble) occurs in any zone in the selected “Start” ZONE LIST (01-15), activation of the selected device will START.

  - ZONE # A specific zone going into alarm, fault, trouble, or restore (Event programming) can be used to start the relay action. Enter the 3-digit zone number (000-128).

3)	ZONE TYPE/SYSTEM OPERATION (ZT).  If all zones to be used to start the device have the same response type, and there are no other zones of this type that are not to activate this device, then “ZONE TYPE” may be used instead of a "ZONE LIST" and "EVENT” to activate the device.

If a system operation, such as “DISARMING” or “ANY FIRE ALARM,” is to activate the device, enter the appropriate choice under the “ZONE TYPE” option.

The “ZONE TYPE/SYSTEM OPERATION” option functions independently of the “EVENT/ZONE LIST” combination.

If a specific "ZONE TYPE" is chosen, any zone of that response type going into alarm, trouble, or fault will cause the device to activate as selected in "ACTION.” If the same “ZONE TYPE” is also chosen for the STOP programming, any zone of that type that restores will de-activate the device.

If a "SYSTEM OPERATION" is chosen, that operation will cause the device to activate as selected in "ACTION.”  The different choices for "ZONE TYPE" and "SYSTEM OPERATION" are listed in “Programming Relays” later in this section, and on the Programming Form.

- PARTITION NO. (P). The device's "Start" ZONE TYPE/SYSTEM OPERATION may be limited to an occurrence on one partition (1-8), or any partition (0).

STOP (STP):	The "STOP" programming determines when and under what conditions the device is de-activated. The following options are available:

1).	RESTORE ZONE LIST (ZL). If a "ZONE LIST" is used as the “Stop” event, the device de-activates when all the zones in that list restore from a previous fault, trouble, or alarm condition.  This occurs regardless of what is programmed to "START" the device; therefore, a "RESTORE ZONE LIST" is normally only used when a "ZONE LIST" is used to start the device.

2).	ZONE TYPE/SYSTEM OPERATION (ZT). Instead of using a "RESTORE ZONE LIST," you can select a specific zone (response) type or system operation action to de-activate the device.

If you choose a specific "ZONE TYPE," any zone of that response type that restores from a previous alarm, trouble, or fault condition will cause the device to de-activate.

If you choose a "SYSTEM OPERATION," that operation causes the device to de-activate.  The different choices for "ZONE TYPE" and "SYSTEM OPERATION" are listed in “Programming Relays” later in this section, and in the Programming Form.

3)	PARTITION NO. (P). The device's "Stop" Zone Type/System Operation may be limited to an occurrence on one partition (1-8), or on any partition (0).

The “ZONE TYPE/SYSTEM OPERATION” option functions independently of the “RESTORE/ZONE LIST” combination.

#### Output Devices Programming

From Data Field Programming Mode, press #93 to display the "ZONE PROG?" prompt.  Press [0] (NO) to each menu option until the "OUTPUT PGM?" prompt appears. Press [1] (YES).

While in this mode, press [] to advance to next screen. Press [#] to back up to the previous screen.

| PROMPT | EXPLANATION |
| --- | --- |
| Enter Relay # (00=Quit) 01 | Enter the relay (output device) identification number 01-96. This is a reference number only, used for identification purposes. The actual module address and relay number on the module are programmed in the last two prompts. Press [] to continue. |
| 02 A EV ZL ZT P STT 0 0 00 00 0 | Press [] to continue. |
| 02 A ZL ZT P STOP 0 00 00 0 | The keypad displays a summary STOP screen. Press [] to continue. |
| 02 Relay Action No Response 0 | The Relay Action is the way in which the relay will respond when activated by the "start" event. Enter the desired action for this relay as follows: 0=not used; 1=close for 2 secs.; 2=stay closed; 3=pulse on/off; 4= toggle on and off alternately |
| 02 Start Event Not used 0 | An output may be activated by an Event/Zone List combination, and/or by a Zone Type/System Operation. For an Event/Zone List combination, enter the event code as follows: 0=not used; 1=alarm; 2=fault; 3=trouble If you are not using a Zone List to activate the relay, enter 0. Press [] to continue. |
| 02 Start: Zn LIST 00 | A zone list is a set of zones that can be used to initiate the start or stop relay action. If a zone list is being used to start this relay action, enter the zone list number,1-15. If a zone list is not being used, enter 0. Press [] to continue. |
| 02 START: ZONE # 000 | A specific zone can be used instead of or in addition to an Event/Zone List or Zone Type/System Operation combination to start the relay action. Enter the 3-digit zone number. Press [] to continue. |
| 02 Start: Zn TypE NO RESPONSE 00 | A Zone Type/System Operation can be used instead of or in addition to an Event/Zone List combination or a specific zone to start the relay action. If a Zone Type/System Operation is being used, enter the 2-digit code as listed in the table that follows. Press [] to continue. |

Choices for Start/Stop Zone Types and System Operations:

| 00 = No Response (Not Used) | 22 = Disarming (Code + Off) | 40 = Bypassing |
| --- | --- | --- |
| 01 = Entry/Exit #1 | 23 = No Alarm Response | 41 = AC Power Fail |
| 02 = Entry/exit #2 | 26 = Verified Alarm | 42 = System Battery Low |
| 03 = Perimeter | 27 = Access Point | 43 = Communication Failure |
| 04 = Interior Follower | 28 = MLB Supervision | 44 = RF Low Battery |
| 05 = Trouble Day/Alarm Night | 29 = Momentary Exit | 45 = Polling Loop Failure |
| 06 = 24-Hr. Silent | 31 = End of Exit Time | 51 = RF Receiver Failure |
| 07 = 24-Hr. Audible | 32 = Start of Entry Time | 52 = Kissoff |
| 08 = 24-Hr. Auxiliary | 33 = Any Burglary Alarm | 54 = Fire Zone Reset |
| 09 = Fire Alarm or Trouble | 34 = Code + [#] + 71 Key Entry | 55 = Disarm + 1 Minute |
| 10 = Interior W/Delay | 35 = Code + [#] + 72 Key Entry | 56 = XX Minutes (enter XX in field 1*74) * |
| 14 = PLM Supervision | 36 = At Bell Timeout ** | 57 = YY Seconds (enter YY in field 1*75) * |
| 19 = 24-Hour Trouble | 37 = 2 Times Bell Timeout ** | 58 = Duress |
| 20 = Arming-STAY | 38 = Chime | 60 = Audio/Video Alarm Verification (must be selected for both START and STOP operation) |
| 21 = Arming-AWAY | 39 = Fire Alarm |  |

*	Stop condition only 	**	Or at disarming, whichever occurs earlier

|  | If using options 56 and/or 57 (usually as the STOP Zone Type), data fields 174 and 175 for the respective relay timeouts for minutes and seconds must be programmed. |  |
| --- | --- | --- |
| PROMPT |  | EXPLANATION |
| 02 Start: ACS PT (00-31) 00 |  | If the start zone type you selected was 27 (access point), this prompt is displayed. Enter the access point (00-31) that will start the relay action. Press [] to continue. |
| 02 Start: PartN ANY PARTITION 0 |  | If the starting event will be limited to occurring on a specific partition, enter the partition number (1-8) in which the start event will occur. Enter 0 for any partition. Press [] to continue. |
|  | Do not use a zone programmed with an RF Button (Input Type BR) to STOP a relay. The system will not de-activate the relay. |  |
| PROMPT |  | EXPLANATION |
| 02 Stop: Zn LIST 00 |  | If a zone list is being used to stop this relay action, enter the zone list number, 1-15. The restore of a zone on the zone list stops the relay. If a zone list is not being used, enter 0. Press [] to continue. |
| 02 Stop: Zn TypE NO RESPONSE 00 |  | If a Zone Type/System Operation is being used to stop the relay action, enter the 2-digit code listed in the Choices for Start/Stop System Operation chart. Press [] to continue. |
| 02 Stop: ACS PT (00-31) 00 |  | If the stop zone type you selected was 27 (access point), this prompt is displayed. Enter the access point (00-31), which will stop the relay action. Press [] to continue. |
| 02 Stop: PartN ANY PARTITION 0 |  | This is the partition to which the stop condition will be limited. Enter 0 for any partition. Enter 1-8 for specific partition number. Press [] to continue. |
| 02 Relay Group 00 |  | Relays may be grouped for common activation by time-driven events (commands 06-10). Enter 00 (no group) or 01-15 for a specific group number. Press [] to continue. |
| 02 Restriction 1=yes 0=no 0 |  | The system may have some devices that are not intended to be under end user control, such as relays activating fire doors or machinery. Enter 1 if the end user will be restricted from accessing this relay group. Press [] to continue. |
| 02 RELAY TYPE V-PLEX 0 |  | Enter 0 for V-Plex (polling loop) devices. Enter 1 for (ECP) relay modules (4204/4204CF). Enter 2 for (X10) Powerline Carrier devices. Enter 3 for (FSA) Fire System Annunciator. Press [] to continue. |
| 02 V-PLEX ZONE # 000 |  | For polling loop trigger outputs (4101SN), enter the protection zone number (010-128) linked to each output, if used. Be sure to enroll the module’s serial number (see Zone Programming). Press [] to continue. |
| 02 ECP ADDRESS 00 |  | If you selected 1 or 3 for (4204/4204CF or FSA), enter the actual module's address (01-15 – 4204/4204CF) (08-23 – FSA)as set by its DIP switches. Up to 8 4204/4204CF and up to 4 FSA modules can be installed in a system. Press [] to continue. |
| 02 MODULE RELAY# 0 |  | For 4204 Relay Outputs, enter the specific relay number on that module (1-4). For 4204CF outputs, enter only module relay number 1 (Output A) or relay number 3 (Output B). These are the only two programmable relays on the 4204CF Module. Press [] to continue. The keypad will display the Start and Stop summary screens again. Press [] to continue. |
| PROMPT |  | EXPLANATION |
| 02 HOUSE CODE A 00 |  | If you selected 2 for Powerline Carrier devices, enter the numerical equivalent of the House Code of the device, as follows: A=00 D=03 G=06 J=09 M=12 P=15 B=01 E=04 H=07 K=10 N=13 C=02 F=05 I=08 L=11 O=14 |
| 02 UNIT CODE 00 |  | Enter the numerical unit code of the Powerline Carrier device (00-15). Press [] to continue. The keypad displays the Start and Stop summary screens again. Press [] to continue. |
| 01 LED # 00 |  | Enter the LED number on the FSA module (01-08 for FSA-8; 01-24 for FSA-24). Press [] to continue. The keypad displays the Start and Stop summary screens again. Press [] to continue. |

If defining a Zone List, continue to the next procedure in this section.  If not, enter 00 +  at the next two prompts.  Then enter 99 to exit programming completely.  After exiting, enter the [Installer Code] + OFF to cancel the system's settling delay.

If supervision of the 4204CF Relay Output is desired, enter a response type for the relay's corresponding supervisory zone. This is equal to 6 + 2-digit relay number. For example, if you are programming relay no. 1, the relay's supervisory zone would be 601. Program this zone for response type 19 (24 hr. trouble) or 05 (trouble by day, alarm by night) in #93 Programming Mode.

#### Zone List Programming

After all relays have been programmed, upon entering 00 at the "ENTER RELAY NO." prompt, you will be asked to enter a Zone List.  If a Zone List number was used to start/stop a relay, you must define the zones belonging to that list.

| PROMPT | EXPLANATION |
| --- | --- |
| ENTER Zn LIST ? 00=QUIT 00 | Enter the zone list number 01-15. Enter 00 to quit. |
| 01 ADD ZONE # 000=QUIT 000 | Using 3-digit entries, enter each zone to be included in this zone list. Press [] after you enter each zone number. When you have entered all zones, enter 000. Press [] to continue. |
| 01 Del Zn LIST ? 1=yes 0=no 0 | Enter 0 to save this zone list. Enter 1 to delete it. |
| 01 DEL ZONES ? 1=yes 0=no 0 | Enter 1 to delete one or more zones in that zone list. Enter 0 if no changes are necessary. If you enter 1, the following screen appears; otherwise, the "Enter Zone LIST" prompt reappears. |
| 01 Zn to DELETE ? 000=QUIT 000 | Enter each zone number to be deleted from the zone list, pressing [] after each number. |
| VIEW Zn LIST ? 00=QUIT 00 | This display appears if you pressed 00 at the “Enter Zone LIST” prompt. Enter the zone list number that you wish to view (01-15). Press [] to continue. |
| 01 ASSIGNED ZONE 000=QUIT 000 | Press [] to scroll through all zones in that list. Enter 000 +[] to quit. Press [1] to exit Menu Mode. Press 99 to exit Program Mode. |

### Relay Voice Descriptors

If you are using the 4286 VIP Module*, voice descriptors can be programmed for outputs 1-96.  These descriptors are announced by the voice module when you access the relays via the # 70 Relay Access Mode over the telephone.

*Note: Does not apply to the usage of TeleCommand.

Each voice descriptor can consist of up to 3 words selected from the Relay Voice Descriptors and Custom Word Substitutes Vocabulary list (later in this section).

|  | The index numbers from this vocabulary list are to be used for relay voice descriptors only. For normal system voice annunciation (e.g., alarms, troubles, status), use the highlighted words in the alpha vocabulary list in the Alpha Programming part of this guide. |
| --- | --- |

To enter relay voice descriptors, do the following:

- From Data Field Programming mode, press #93 to display the "ZONE PROG?" prompt.

- Press [0] (NO) to each menu option until the "RLY VOICE DESCR?" prompt is displayed.  Follow the instructions below.  While in this mode, press [] to advance to next screen. Press [] to back up to previous screen.

| PROMPT | EXPLANATION |
| --- | --- |
| RLY VOICE DESCR? 1=yes 0=no 0 | Press [1] to program voice descriptors for relays. |
| ENTER RELAY NO. 00=QUIT 01 | Enter the 2-digit relay number (01-32) for the relay desired, or enter 00 to quit Relay Voice Descriptor Programming Mode. Press [] |
| 01 ENTER DESC d1 | From the Relay Voice Descriptors and Custom Word Substitutes Vocabulary list, enter the 3-digit index number for the first word of the relay descriptor phrase. Press [] to accept entry. |
| 01 ENTER DESC d2 | From the Relay Voice Descriptors and Custom Word Substitutes Vocabulary list, enter the 3-digit index number for the second word of the relay descriptor phrase. If second word is not desired, press [000]. Press [] to accept entry. |
| 01 ENTER DESC d3 | From the Relay Voice Descriptors and Custom Word Substitutes Vocabulary list, enter the 3-digit index number for the third word of the relay descriptor phrase. If third word is not desired, press [000]. Press [] to accept entry. The “ENTER RELAY NO.” prompt appears. Enter the next relay number to be programmed. When you have programmed all output devices, enter 00 to quit. Enter 99 to exit Program Mode. |

### Relay Voice Descriptors and Custom Word Substitutes Vocabulary

Word	Index

Air	116

Alarm	255

And	067

Apartment	117

Appliances	161

Area	118

Attic	119

Baby	120

Back	121

Bar	122

Basement	021

Bathroom	051

Battery	053

Bed	092

Bedroom	015

Blower	123

Boiler	124

Bright	162

Building	125

Burglary	039

Call	009

Central	089

Chime	054

Closed	126

Computer	127

Console	066

Word	Index

Daughter's	208

Den	052

Detector	128

Device	060

Dim	163

Dining	031

Door	016

Down	008

Downstairs	184

Driveway	130

Duct	131

East	132

Eight	077

Eighth	221

Equipment	133

Exit	004

Factory	134

Father's	211

Fence	135

Fifth	218

Fire	040

First 	136

Five	074

Floor	029

Four	073

Fourth	217

Foyer	137

Word	Index

Front	087

Garage	023

Gas	138

Glass	139

Hall	050

Heat	010

Inside	209

Kitchen	022

Laundry	140

Left	027

Library	141

Light	019

Living	030

Loading	142

Lower	094

Machine	143

Master	144

Medical	014

Mother's	212

Motion	145

Nine	078

Ninth	222

Word	Index

No	165

North	146

Not	012

Off	011

Office	147

On	058

One	070

Open	148

Outside	210

Panic	013

Partition	090

Patio	149

Phone	061

Power	063

Pump	166

Rear 	088

Right	028

Room	018

's	007

Second	056

Service	150

Seven	076

Seventh	220

Shed	151

Shop	152

Word	Index

Side	153

Six	075

Sixth	219

Smoke	024

Son's	223

South	155

Stairs	006

Station	156

Storage	157

Sun	154

System	062

Temperature	158

Third	159

Three	072

Tool	213

Two	071

Up	025

Upper	187

Upstairs	183

Utility	185

West	215

Window	017

Wing	216

Zero	069

Zone	002

### Custom Word Substitutes for VIP Module Annunciation

A substitute word can be programmed for each of the 60 custom words used in your alpha zone descriptions.  The VIP Module announces this substitute word in place of the custom word that is displayed on the alpha keypad.  For example, an alarm display of "John's Bedroom" could be announced as "Son's Bedroom," as there is no annunciation for the custom word "John."  Note that if a substitute word is not assigned, the VIP Module will not annunciate the zone descriptor at all, but will only annunciate the zone number.

To enter custom word substitutes, do the following:

- From Data Field Programming Mode, press #93 to display the "ZONE PROG?" prompt.

- Press [0] (NO) to each menu option until the "CUSTOM INDEX ?" prompt is displayed.

| PROMPT | EXPLANATION |  |
| --- | --- | --- |
| CUSTOM INDEX ? 1=yes 0=no 0 |  | Enter [1] at this prompt. |
| CUSTOM WORD NO. 00=QUIT 00 |  | Enter the custom word number (01-60) for which a voice substitute is desired. Enter 00 to quit this Programming Mode. Press [] to accept entry. |
| 01 ENTER INDEX # |  | Enter the 3-digit substitute word index number from the Relay Voice Descriptors and Custom Word Substitutes Vocabulary list in the Relay Voice Descriptors part of this section. Press [] to accept entry. The "CUSTOM WORD NO." prompt is displayed. Enter the next custom word number to be substituted, or enter 00 to quit. |

## System Layout Worksheets

Before programming any security system, you should first define the installation. To help you lay out a partitioned system, use the following worksheets. This will further simplify the programming process.

| PARTITIONS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Partition # | Descriptor (4 char max) | Primary Sub. # | Secondary Sub. # | Alpha Default Message (32 character maximum) |
| Partition 1 |  |  |  |  |
| Partition 2 |  |  |  |  |
| Partition 3 |  |  |  |  |
| Partition 4 |  |  |  |  |
| Partition 5 |  |  |  |  |
| Partition 6 |  |  |  |  |
| Partition 7 |  |  |  |  |
| Partition 8 |  |  |  |  |
| Zone 7 Keyswitch Arming Partition Assignment (1-8): |  |  |  |  |
| Wireless Keypad Partition Assignment (1-8): |  |  |  |  |
| Voice Module Partition Assignment (1-8): |  |  |  |  |
| Use Partition Descriptor (yes/no)? |  |  |  |  |
| Common Area 1 Partition Assignment (1-8): |  |  |  |  |
| Common Area 2 Partition Assignment (1-8): |  |  |  |  |
| Common Area 3 Partition Assignment (1-8): |  |  |  |  |

| COMMUNICATION OPTIONS BY PARTITION (enter yes/no) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Option | Part. 1 | Part. 2 | Part. 3 | Part. 4 | Part. 5 | Part. 6 | Part. 7 | Part. 8 |
| Intermittent Sensor Suppression Count (00-15; 00 = no suppression |  |  |  |  |  |  |  |  |
| Cancel Report After Disarm |  |  |  |  |  |  |  |  |
| Cancel Report After Disarm |  |  |  |  |  |  |  |  |
| Dialer Reports for Panic ( + 1) |  |  |  |  |  |  |  |  |
| Dialer Reports for Panic (# + 3) |  |  |  |  |  |  |  |  |
| Dialer Reports for Panic ( + #) |  |  |  |  |  |  |  |  |
| Dialer Reports for Duress |  |  |  |  |  |  |  |  |
| Burglary Alarm Comm. Delay (16 sec.) |  |  |  |  |  |  |  |  |
| Entry Delay #1 (15-225 seconds) |  |  |  |  |  |  |  |  |
| Exit Delay #1 (15-225 seconds) |  |  |  |  |  |  |  |  |
| Entry Delay #2 (15-225 seconds) |  |  |  |  |  |  |  |  |
| Exit Delay #2 (15-225 seconds) |  |  |  |  |  |  |  |  |
| Quick Arming |  |  |  |  |  |  |  |  |
| Multiple Alarms per Arming |  |  |  |  |  |  |  |  |

| SYSTEM DEFINITIONS BY PARTITION (enter values or yes/no) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Option | Part. 1 | Part. 2 | Part. 3 | Part. 4 | Part. 5 | Part. 6 | Part. 7 | Part. 8 |
| Keypad Panic for zone 995 ( + 1) |  |  |  |  |  |  |  |  |
| Keypad Panic for zone 996 (# + 3) |  |  |  |  |  |  |  |  |
| Keypad Panic for zone 999 ( + #) |  |  |  |  |  |  |  |  |
| Allow Sign-on (GOTO function) |  |  |  |  |  |  |  |  |
| Non-Bypassable Zone* |  |  |  |  |  |  |  |  |
| Sounder Timeout for Siren (2 min. increments) |  |  |  |  |  |  |  |  |
| Keypad Annunciation During Entry** |  |  |  |  |  |  |  |  |
| Keypad Annunciation During Exit |  |  |  |  |  |  |  |  |
| Confirmation of Arming Ding for Siren |  |  |  |  |  |  |  |  |
| Chime on Siren |  |  |  |  |  |  |  |  |
| Access Control Relay (field 176) |  |  |  |  |  |  |  |  |
| Affects Common Area 1 (check all that apply) |  |  |  |  |  |  |  |  |
| Arms Common Area 1 (check all that apply) |  |  |  |  |  |  |  |  |
| Affects Common Area 2 (check all that apply) |  |  |  |  |  |  |  |  |
| Arms Common Area 2 (check all that apply) |  |  |  |  |  |  |  |  |
| Affects Common Area 3 (check all that apply) |  |  |  |  |  |  |  |  |
| Arms Common Area 3 (check all that apply) |  |  |  |  |  |  |  |  |
| Displays Fire Alarms of Other Partitions |  |  |  |  |  |  |  |  |
| Displays Burg & Panic Alarms of Other Part. |  |  |  |  |  |  |  |  |
| Displays Troubles of Other Partitions |  |  |  |  |  |  |  |  |
| *Can be any zone 1-128 **no = 3 beeps yes = continuous |  |  |  |  |  |  |  |  |

| PRINTER OPTIONS |  |  | EVENT LOG TYPES |  |  |
| --- | --- | --- | --- | --- | --- |
| 2 or 24 hour Time format |  |  | Option | No | Yes |
| Printer On-Line (yes/no) |  |  | Alarm |  |  |
| 1200 or 300 baud Printer Baud Rate |  |  | Trouble |  |  |
|  |  |  | Bypass |  |  |
|  |  |  | Open/Close |  |  |
|  |  |  | System |  |  |
|  |  |  | Test Report |  |  |

| DEVICES (keypads, 4204/4204CF, RF receivers, VIP module, VGM, PLM) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addr | Type | Part | Sound Opt | Supv CF? | House ID | Kypd Glbal | AUI |  |
|  |  |  |  |  |  |  |  | Device Types: 00 = Device Not Used 01 = Alpha Keypad or AUI 02 = Fixed Word Keypad or TeleCommand 03 = RF Receiver 04 = Output Relay Module 05 = 4286 Voice Module 06 = LRR, TCP-IP, or Contact ID on Keypad Bus 08 = Fire Display Module 09 = Vista Gateway Module 10 = Panel Link Module NOTES: Address 04 must be used for the 4286 Voice Module, if used. Console Sounder Options: 0 = No Suppression 1 = Suppress Arm/Disarm and Entry/Exit Beeps 2 = Suppress Chime Mode Beeps Only 3 = Suppress Arm/Disarm, Entry/Exit and Chime Mode Beeps Defaults: Addresses 00 = Alpha Keypad; No Suppression |

| ACCESS CODES & USER DEFINITIONS FOR PARTITIONS 1 & 2 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4-digit Security Code | Access Group 0; 1-8 | Partition 1 |  |  |  |  | Partition 2 |  |  |  |  |
|  |  | 3-Digit User # | Auth. Level | Open/ Close | RF Key | Global Arm | 3-Digit User # | Auth. Level | Open/ Close | RF Key | Global Arm |

| ACCESS CODES & USER DEFINITIONS FOR PARTITIONS 3 & 4 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4-digit Security Code | Access Group 0; 1-8 | Partition 3 |  |  |  |  | Partition 4 |  |  |  |  |
|  |  | 3-Digit User # | Auth. Level | Open/ Close | RF Key | Global Arm | 3-Digit User # | Auth. Level | Open/ Close | RF Key | Global Arm |

| ACCESS CODES & USER DEFINITIONS FOR PARTITIONS 5 & 6 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4-digit Security Code | Access Group 0; 1-8 | Partition 5 |  |  |  |  | Partition 6 |  |  |  |  |
|  |  | 3-Digit User # | Auth. Level | Open/ Close | RF Key | Global Arm | 3-Digit User # | Auth. Level | Open/ Close | RF Key | Global Arm |

| ACCESS CODES & USER DEFINITIONS FOR PARTITIONS 7 & 8 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4-digit Security Code | Access Group 0; 1-8 | Partition 7 |  |  |  |  | Partition 8 |  |  |  |  |
|  |  | RF Key | Auth. Level | Open/ Close | RF Key | Global Arm | 3-Digit User # | Auth. Level | Open/ Close | RF Key | Global Arm |

Authority Levels:	1 = master (arm, disarm, bypass, and/or modify lower level users)

2 = manager (arm, disarm, bypass, and/or modify lower level users)

3 = operator A (arm, disarm, bypass)

4 = operator B (arm, disarm)

5 = operator C (arm, disarm only if system was armed with this code)

6 = duress code (arm, disarm, triggers silent panic alarm

| ZONE DEFINITIONS FOR ZONES 1-25 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Zone Number | Zone Type | Part. (1-8) | Input Type | Serial Number/Loop | Report Code | Zone Information (part numbers) and Alpha Descriptor (3 words max.) |

| ZONE DEFINITIONS FOR ZONES 26-50 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Zone Number | Zone Type | Part. (1-8) | Input Type | Serial Number/Loop | Report Code | Zone Information (part numbers) and Alpha Descriptor (3 words max.) |

| ZONE DEFINITIONS FOR ZONES 51-75 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Zone Number | Zone Type | Part. (1-8) | Input Type | Serial Number/Loop | Report Code | Zone Information (part numbers) and Alpha Descriptor (3 words max.) |

| ZONE DEFINITIONS FOR ZONES 76-100 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Zone Number | Zone Type | Part. (1-8) | Input Type | Serial Number/Loop | Report Code | Zone Information (part numbers) and Alpha Descriptor (3 words max.) |

| ZONE DEFINITIONS FOR ZONES 101-125 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Zone Number | Zone Type | Part. (1-8) | Input Type | Serial Number/Loop | Report Code | Zone Information (part numbers) and Alpha Descriptor (3 words max.) |

| ZONE DEFINITIONS FOR ZONES 126-128 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Zone Number | Zone Type | Part. (1-8) | Input Type | Serial Number/Loop | Report Code | Zone Information (part numbers) and Alpha Descriptor (3 words max.) |

| ZONE DEFINITIONS FOR KEYPAD PANIC ZONES 995, 996, & 999 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Enter yes/no for each partition-field *22 |  |  |  |  |  |  |  |  |  |
| Zone No. | Zone Type | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | Report Code | Zone Information (part numbers) & Alpha Descriptor (3 words max.) |
| 995 |  |  |  |  |  |  |  |  |  |  |  |
| 996 |  |  |  |  |  |  |  |  |  |  |  |
| 999 |  |  |  |  |  |  |  |  |  |  |  |

NOTE: Zones 601 through 830, 970, 988, 990, and 997 must be supervised and programmed with response type 05 (Day/Night) to comply with the “EN50131” specification.

| ZONE DEFINITIONS FOR SYSTEM ZONES; 988; 990 & 997 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Zone No. | Zone Type | Report Code | Zone Information (part numbers) & Alpha Descriptor (3 words max.) | (Notes) |
| 970 |  |  |  | (Bell Supervision) |
| 988 |  |  |  | (RF Rcvr #2 Supv) |
| 990 |  |  |  | (RF Rcvr #1 Supv) |
| 997 |  |  |  | (Polling Loop Short) |

| ZONE DEFINITIONS FOR RELAY SUPERVISORY ZONES 601-696 NOTE: Only the relays on 4204CF modules may be supervised. If supervision is programmed for other types of Output Devices, unpredictable results may occur. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Zone No. | Zone Type | Report Code | Zone Info (part numbers) & Alpha Descriptor (3 words max.) | Zone No. | Zone Type | Report Code | Zone Infor (part numbers) & Alpha Descriptor (3 words max.) |

| ZONE DEFINITIONS FOR RELAY SUPERVISORY ZONES 601-696 (cont’d) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Zone No. | Zone Type | Report Code | Zone Info (part numbers) & Alpha Descriptor (3 words max.) | Zone No. | Zone Type | Report Code | Zone Infor (part numbers) & Alpha Descriptor (3 words max.) |

| ZONE DEFINITIONS FOR SUPERVISORY OF ECP DEVICE ZONES 800-830 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Zone No. | Zone Type | Report Code | Zone Info (part numbers) & Alpha Descriptor (3 words max.) | Zone No. | Zone Type | Report Code | Zone Infor (part numbers) & Alpha Descriptor (3 words max.) |
| 800 |  |  |  | 816 |  |  |  |
| 801 |  |  |  | 817 |  |  |  |
| 802 |  |  |  | 818 |  |  |  |
| 803 |  |  |  | 819 |  |  |  |
| 804 |  |  |  | 820 |  |  |  |
| 805 |  |  |  | 821 |  |  |  |
| 806 |  |  |  | 822 |  |  |  |
| 807 |  |  |  | 823 |  |  |  |
| 808 |  |  |  | 824 |  |  |  |
| 809 |  |  |  | 825 |  |  |  |
| 810 |  |  |  | 826 |  |  |  |
| 811 |  |  |  | 827 |  |  |  |
| 812 |  |  |  | 828 |  |  |  |
| 813 |  |  |  | 829 |  |  |  |
| 814 |  |  |  | 830 |  |  |  |
| 815 |  |  |  |  |  |  |  |

Zone Types:

| 00=zone not used | 07=24-hour audible | 21=arm away |
| --- | --- | --- |
| 01=entry/exit 1 | 08=24-hour auxiliary | 22=disarm |
| 02=entry/exit 2 | 09=supervised fire | 23=no alarm response |
| 03=perimeter | 10=interior (delay) | 27=access control |
| 04=interior (follower) | 14=PLM supervision | 28=MLB supervision |
| 05=day/night burglary | 20=arm stay | 29=momentary exit |
| 06=24-hour silent |  |  |

Input Types:

| 00=not used | 07=DIP switch-type polling loop |
| --- | --- |
| 01=hardwired | 08=right loop DIP switch polling loop |
| 02=RF motion transmitter | 09=keypad input |
| 03=supervised RF transmitter | 10=PassPoint ACS input |
| 04=unsupervised RF transmitter | 11=VistaKey door status monitor |
| 05=RF button transmitter | 12=VistaKey request to exit |
| 06=serial number polling loop | 13=VistaKey general purpose |

- Output Devices Worksheets

- Applicable only if relays (4204/4204CF), FSA Modules, V-Plex or Powerline Carrier Devices (X-10) are used.

Output Devices – Programmed in the #93 Menu Mode in the Output Programming Section. Fill in the required data on the worksheet below and follow the procedure in the installation instructions as you enter the data during the displays and prompts that appear in sequence.

Notes:	1.	For 4204/4204CF and FSA, the Device Programming section must be programmed for the device address. Set the DIP switches on the device for that address.

2.	For V-Plex, devices must be programmed in the Zone Programming section.

3.	For Powerline Carrier Devices (X-10), use the 4300 or XF10 transformer in place of the transformer that comes in the box with the control panel.

| OUTPUT DEV # | A | START |  |  |  |  | STOP |  |  | Relay Group | Restrict | 0=V-Plex 1=4204 2=X-10 3=FSA | V-Plex Zone # or Dev Add 4204 or FSA or House Code for X-10 | Relay # for 4204 or LED # for FSA or Unit Code for X-10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | EV/ZL |  | Zone | ZT / P |  | ZL | ZT / P |  |  |  |  |  |  |

- OUTPUT DEVICES WORKSHEET (cont’d)

| OUTPUT DEV # | A | START |  |  |  |  | STOP |  |  | Relay Group | Restrict | 0=V-Plex 1=4204 2=X-10 3=FSA | V-Plex Zone # or Dev Add 4204 or FSA or House Code for X-10 | Relay # for 4204 or LED # for FSA or Unit Code for X-10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | EV/ZL |  | Zone | ZT / P |  | ZL | ZT / P |  |  |  |  |  |  |

OUTPUT DEVICES WORKSHEET (cont’d)

| OUTPUT DEV # | A | START |  |  |  |  | STOP |  |  | Relay Group | Restrict | 0=V-Plex 1=4204 2=X-10 3=FSA | V-Plex Zone # or Dev Add 4204 or FSA or House Code for X-10 | Relay # for 4204 or LED # for FSA or Unit Code for X-10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | EV/ZL |  | Zone | ZT / P |  | ZL | ZT / P |  |  |  |  |  |  |

OUTPUT DEVICES WORKSHEET (cont’d)

| OUTPUT DEV # | A | START |  |  |  |  | STOP |  |  | Relay Group | Restrict | 0=V-Plex 1=4204 2=X-10 3=FSA | V-Plex Zone # or Dev Add 4204 or FSA or House Code for X-10 | Relay # for 4204 or LED # for FSA or Unit Code for X-10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | EV/ZL |  | Zone | ZT / P |  | ZL | ZT / P |  |  |  |  |  |  |

A  =	DEVICE ACTION	0 = No Response; 1 = Close for 2 sec; 2 = Close and stay closed; 3 = Pulse on and off;

4 = Toggle alternately between START and STOP events

EV =	EVENT	0 = Not used; 1 = Alarm; 2 = Fault; 3 = Trouble; 4 = Restore

ZL  =	ZONE LIST	01-15, 00 = Not Used

"START" ZONE LIST:	Upon alarm, fault, trouble or restore of ANY zone in this list, device action will START.

"STOP" ZONE LIST:	Upon restore of ALL zones on this list, device action will STOP.  It need not be same list as used for START.

ZT =	ZONE TYPE/SYSTEM OPERATION

- Choices for Zone Types are:

| 00=zone not used | 06=24-hour silent | 21=arm away |
| --- | --- | --- |
| 01=entry/exit 1 | 07=24-hour audible | 22=disarm |
| 02=entry/exit 2 | 08=24-hour auxiliary | 23=no alarm response |
| 03=perimeter | 09=supervised fire | 27=access control |
| 04=interior (follower) | 10=interior (delay) | 28=MLB supervision |
| 05=day/night burglary | 20=arm stay | 29=momentary exit |

NOTE:	Any zone in "ZT" for Start, going into alarm, fault, or trouble will activate the relay.

Any zone in "ZT" for Stop, that restores will stop the relay action.

- Choices for System Operation are:

| 00 = No Response (Not Used) | 27 = Access Point (allows more than one relay to be controlled by activation if access point request) | 42 = System Battery Low |
| --- | --- | --- |
| 01 = Entry/Exit #1 |  | 43 = Communication Failure |
| 02 = Entry/exit #2 |  | 44 = RF Low Battery |
| 03 = Perimeter | 28 = MLB Supervision | 45 = Polling Loop Failure |
| 04 = Interior Follower | 29 = Momentary Exit | 47 = Console Failure |
| 05 = Trouble Day/Alarm Night | 31 = End of Exit Time | 51 = RF Receiver Failure |
| 06 = 24-Hr. Silent | 32 = Start of Entry Time | 52 = Kissoff |
| 07 = 24-Hr. Audible | 33 = Any Burglary Alarm | 54 = Fire Zone Reset |
| 08 = 24-Hr. Auxiliary | 34 = Code + [#] + 71 Key Entry | 55 = Disarm + 1 Minute |
| 09 = Fire Alarm or Trouble | 35 = Code + [#] + 72 Key Entry | 56 = XX Minutes (enter XX in field 1*74) * |
| 10 = Interior w/Delay | 36 = At Bell Timeout ** | 57 = YY Seconds (enter YY in field 1*75) * |
| 20 = Arming-STAY | 37 = 2 Times Bell Timeout ** | 58 = Duress |
| 21 = Arming-AWAY | 38 = Chime | 60 = Audio Alarm Verification (must be selected for both START and STOP operation) |
| 22 = Disarming (Code + Off) | 39 = Fire Alarm |  |
| 23 = No Alarm Response | 40 = Bypassing |  |
| 26 = Verified Alarm | 41 = AC Power Fail |  |

*	Stop condition only

**	Or at Disarming, whichever occurs earlier

P =	PARTITION No.  1-8, 0 = Any

ZONE LISTS FOR OUTPUT DEVICES – Programmed in the #93 Menu Mode in the Output Programming Section. Fill in the required data on the worksheet below and follow the procedure shown earlier in this Programming Guide as you enter the data during the displays and prompts that appear in sequence. Up to 15 zone lists may be created

Note:	Record desired zone numbers below. More or fewer boxes than shown may be needed, as any list may include any or all of system's zone numbers.

Zone List 1: Started or stopped by zone numbers (enter 000 to end entries).

Zone List 2: Started or stopped by zone numbers (enter 000 to end entries).

Zone List 3: Started or stopped by zone numbers (enter 000 to end entries).

Zone List 4: Started or stopped by zone numbers (enter 000 to end entries).

Zone List 5: Started or stopped by zone numbers (enter 000 to end entries).

Zone List 6: Started or stopped by zone numbers (enter 000 to end entries).

Zone List 7: Started or stopped by zone numbers (enter 000 to end entries).

Zone List 8: Started or stopped by zone numbers (enter 000 to end entries).

Zone List 9: Started or stopped by zone numbers (enter 000 to end entries).

Zone List 10: Started or stopped by zone numbers (enter 000 to end entries).

Zone List 11: Started or stopped by zone numbers (enter 000 to end entries).

Zone List 12: Started or stopped by zone numbers (enter 000 to end entries).

Zone List 13: Started or stopped by zone numbers (enter 000 to end entries).

Zone List 14: Started or stopped by zone numbers (enter 000 to end entries).

Zone List 15: Started or stopped by zone numbers (enter 000 to end entries).

- Scheduling Menu Prompts

To program schedules, enter Scheduling program mode by pressing [User Code] + # + 80 to display the first choice of the menu driven programming functions.  NOTE: Only users with an Installer or Master level user code may enter the #80 mode.

Press 0 (NO) or 1 (YES) in response to the displayed menu selection. Pressing 0 will display the next choice in sequence. Menu selections are as follows:

| PROMPT | EXPLANATION |
| --- | --- |
| Time Window ? 1 = YES 0 = NO 0 | For defining up to 20 time periods each with a start and a stop time programmed by entering the hours and minutes. |
| O/C Schedules ? 1 = YES 0 = NO 0 | For defining the daily open and close schedules for the 8 partitions. Each partition can be programmed with an opening and closing time period for each day of the week and holidays. |
| Holidays ? 1 = YES 0 = NO 0 | For defining up to 16 holidays for which partitions they apply. |
| Timed Events ? 1 = YES 0 = NO 0 | For defining up to 20 time driven events with the following parameters: - Time period - Action desired - Action specifier - Activation time - Days of the week |
| Access Sched. ? 1 = YES 0 = NO 0 | For defining the limitation of access schedules for the user codes. Each schedule can be programmed with two time period for each day of the week and holidays |

#80 & #81 MENU MODE KEY COMMANDS

The following is a list of commands used while in the Menu mode.

| #80 or #81 | Enters Menu mode |
| --- | --- |
| [Q] | Serves as ENTER key. Press to have keypad accept entry. |
| [#] | Backs up to previous screen. |
| 0 | Press to answer NO. |
| 1 | Press to answer YES. |
| 01-09 | All data entries are either 2-digit entries. |
| 00 | Exits Menu mode, returns to normal operation mode when entered at the first question for each category. |

- Scheduling Worksheets

Time Periods Definitions Worksheet.  The system provides 20 time periods that are defined with start and stop times.  They are programmed in the #80 Menu Mode.  Fill in the required data on the worksheet below and follow the procedure in the installation instructions as you enter the data during the displays and prompts that appear in sequence.

| Time Period Number | Start Time (HH:MM) | Stop Time (HH:MM) |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |
| 7 |  |  |
| 8 |  |  |
| 9 |  |  |
| 10 |  |  |
| 11 |  |  |
| 12 |  |  |
| 13 |  |  |
| 14 |  |  |
| 15 |  |  |
| 16 |  |  |
| 17 |  |  |
| 18 |  |  |
| 19 |  |  |
| 20 |  |  |

(Keep this worksheet handy, as you will be asked for a given time period number later in this section).

|  | Because the time periods are shared among all partitions, it is important to make sure that changing a time period does not adversely affect desired actions in other partitions. |
| --- | --- |

Daily Open/Close Schedule Worksheet:  Using the time periods previously defined, fill in the required data on the worksheet below and follow the procedure in the installation instructions as you enter the data during the displays and prompts that appear in sequence.

| Part | Mon |  | Tues |  | Wed |  | Thur |  | Fri |  | Sat |  | Sun |  | Hol |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Op | Cl | Op | Cl | Op | Cl | Op | Cl | Op | Cl | Op | Cl | Op | Cl | Op | Cl |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Holiday Schedule Worksheet:  The system provides up to 16 holidays that can be assigned for the system. Each holiday can be assigned to any combination of partitions. Fill in the required data on the worksheet below and follow the procedure in the installation instructions as you enter the data during the displays and prompts that appear in sequence.

| HOL | Partition |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Month/Day | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | / |  |  |  |  |  |  |  |  |
| 2 | / |  |  |  |  |  |  |  |  |
| 3 | / |  |  |  |  |  |  |  |  |
| 4 | / |  |  |  |  |  |  |  |  |
| 5 | / |  |  |  |  |  |  |  |  |
| 6 | / |  |  |  |  |  |  |  |  |
| 7 | / |  |  |  |  |  |  |  |  |
| 8 | / |  |  |  |  |  |  |  |  |
| 9 | / |  |  |  |  |  |  |  |  |
| 10 | / |  |  |  |  |  |  |  |  |
| 11 | / |  |  |  |  |  |  |  |  |
| 12 | / |  |  |  |  |  |  |  |  |
| 13 | / |  |  |  |  |  |  |  |  |
| 14 | / |  |  |  |  |  |  |  |  |
| 15 | / |  |  |  |  |  |  |  |  |
| 16 | / |  |  |  |  |  |  |  |  |

Time-Driven Event Worksheet: The system provides up to 20 time-driven events that can be programmed for the system.  Fill in the required data on the worksheet below and follow the procedure in the installation instructions as you enter the data during the displays and prompts that appear in sequence.

| Sched | Time | Day(s) |  |  |  |  |  |  |  | Action | Action | Activation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Num. | Time period | M | T | W | T | F | S | S | H | Desired | Specifier | Time |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |  |  |  |  |  |  |

Below is a list of the "Action" codes (desired actions) used when programming time-driven events. Note that these codes are independent of the "relay codes" programmed during the #93 Menu Mode–Output Programming mode. If using Time Driven Events, the following menu items must first be programmed using #93 Menu Mode - Output Programming:

Enter Relay No.	(reference identification number)

Relay Group	(if applicable)

Restriction

Relay Type	(V-Plex, 4204/4204CF, FSA, or X-10)

Zone No.	(V-Plex)

ECP Address	(4204/4204CF)

Relay No.	(4204/4204CF)

LED No.	(FSA)

House Code	(X-10)

Unit Code	(X-10)

#### Relay commands:

Action Specifier for commands 01-05 is Relay No.; Action Specifier for commands 06-10 is Relay Group No.

01 = Relay On	02 = Relay Off

03 = Relay Close for 2 seconds	04 = Relay Close XX minutes  (field 1*74)

05 = Relay Close YY seconds (field 1*75)	06 = Relay Group On

07 = Relay Group Off	08 = Relay Group Close for 2 seconds

09 = Relay Group Close XX minutes (field 1*74) 	10 = Relay Group Close YY seconds (field 1*75)

#### Arm/Disarm commands:

Action Specifier for commands 20-26 is Partition(s).  Activation times 1 (Beginning), 2 (End), 3 (During), are the only valid choices for auto-arming and disarming functions.

20 = Arm-Stay		21 = Arm Away

22 = Disarm	23 = Force Arm Stay (Auto-bypass faulted zns)

24 = Force Arm Away (Auto-bypass faulted zns)	25 = Arm Instant

26 = Arm Maximum

#### Bypass commands:

Action Specifier for commands 30-31 is Zone List #,  Activation times 1 (Beginning), 2 (End), 3 (During), are the only valid choices for bypass commands.

30 = Auto bypass - Zone list	31 = Auto unbypass - Zone list

#### Open/Close Periods:

Action Specifier for commands 40-41 is Partition(s), and for 42 is Access Group.  Activation time 3 (During), are the only valid choices for these commands.

40 = Enable Opening Time Period 	41 = Enable Closing Time Period	42 = Enable Access Time Period

#### Access Control Commands

Action Specifier for commands 55-60 is Access Point, for 61-66 is Group, for 67-72 is Partition, and for 73-74 is Trigger.

55 = Access Point Grant	56 = Access Point Grant with Override

57 = Access Point Protect	58 = Access Point Bypass

59 = Access Point Lock	60 = Access Point Exit

61 = Access Point Group Grant	62 = Access Point Group Grant with Override

63 = Access Point Group Protect	64 = Access Point Group Bypass

65 = Access Point Group Lock	66 = Access Point Group Exit

67 = Access Point Partition Grant	68 = Access Point Partition Grant with Override

69 = Access Point Protect by Partition	70 = Access Point Bypass by Partition

71 = Access Point Lock by Partition	72 = Access Point Exit by Partition

73 = Access Point Trigger On	74 = Access Point Trigger Off

#### Additional Commands

Action Specifier for command 77-78 is Group.

77 = Access Point Group Enable	78 = Access Point Group Disable

#### Activation time:

Refers to when the action is to take place relative to the time period.

1 =	Beginning of time period

2 =	End of time period

3 =	During time period active period only (On at beginning of time period, off at end).

4 =	Beginning and end of time period

Limitation of Access Worksheet The system provides up to 8 Access Schedules that can be programmed for the system.  Fill in the required data on the worksheet below and follow the procedure in the installation instructions as you enter the data during the displays and prompts that appear in sequence.

| Acc | Mon |  | Tues |  | Wed |  | Thurs |  | Fri |  | Sat |  | Sun |  | Hol |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sch | W1 | W2 | W1 | W2 | W1 | W2 | W1 | W2 | W1 | W2 | W1 | W2 | W1 | W2 | W1 | W2 |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Temporary Schedule #81 Menu Mode. The system provides a Temporary Schedule for each partition.  Enter the temporary scheduling mode by pressing [Installer Code] + [#] + [81].  Fill in the required data on the worksheet below and follow the procedure in the installation instructions as you enter the data during the displays and prompts that appear in sequence.

| Partition/Time Periods |  | Mon | Tue | Wed | Thu | Fri | Sat | Sun |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Disarm Time Period |  |  |  |  |  |  |  |
|  | Start Time HH:MM |  |  |  |  |  |  |  |
|  | Stop Time HH:MM |  |  |  |  |  |  |  |
|  | Arm Time Period |  |  |  |  |  |  |  |
|  | Start Time HH:MM |  |  |  |  |  |  |  |
|  | Stop Time HH:MM |  |  |  |  |  |  |  |
| 2 | Disarm Time Period |  |  |  |  |  |  |  |
|  | Start Time HH:MM |  |  |  |  |  |  |  |
|  | Stop Time HH:MM |  |  |  |  |  |  |  |
|  | Arm Time Period |  |  |  |  |  |  |  |
|  | Start Time HH:MM |  |  |  |  |  |  |  |
|  | Stop Time HH:MM |  |  |  |  |  |  |  |
| Partition/Time Periods |  | Mon | Tue | Wed | Thu | Fri | Sat | Sun |
| 3 | Disarm Time Period |  |  |  |  |  |  |  |
|  | Start Time HH:MM |  |  |  |  |  |  |  |
|  | Stop Time HH:MM |  |  |  |  |  |  |  |
|  | Arm Time Period |  |  |  |  |  |  |  |
|  | Start Time HH:MM |  |  |  |  |  |  |  |
|  | Stop Time HH:MM |  |  |  |  |  |  |  |
| 4 | Disarm Time Period |  |  |  |  |  |  |  |
|  | Start Time HH:MM |  |  |  |  |  |  |  |
|  | Stop Time HH:MM |  |  |  |  |  |  |  |
|  | Arm Time Period |  |  |  |  |  |  |  |
|  | Start Time HH:MM |  |  |  |  |  |  |  |
|  | Stop Time HH:MM |  |  |  |  |  |  |  |
| 5 | Disarm Time Period |  |  |  |  |  |  |  |
|  | Start Time HH:MM |  |  |  |  |  |  |  |
|  | Stop Time HH:MM |  |  |  |  |  |  |  |
|  | Arm Time Period |  |  |  |  |  |  |  |
|  | Start Time HH:MM |  |  |  |  |  |  |  |
|  | Stop Time HH:MM |  |  |  |  |  |  |  |
| 6 | Disarm Time Period |  |  |  |  |  |  |  |
|  | Start Time HH:MM |  |  |  |  |  |  |  |
|  | Stop Time HH:MM |  |  |  |  |  |  |  |
|  | Arm Time Period |  |  |  |  |  |  |  |
|  | Start Time HH:MM |  |  |  |  |  |  |  |
|  | Stop Time HH:MM |  |  |  |  |  |  |  |
| 7 | Disarm Time Period |  |  |  |  |  |  |  |
|  | Start Time HH:MM |  |  |  |  |  |  |  |
|  | Stop Time HH:MM |  |  |  |  |  |  |  |
|  | Arm Time Period |  |  |  |  |  |  |  |
|  | Start Time HH:MM |  |  |  |  |  |  |  |
|  | Stop Time HH:MM |  |  |  |  |  |  |  |

| Partition/Time Periods |  | Mon | Tue | Wed | Thu | Fri | Sat | Sun |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | Disarm Time Period |  |  |  |  |  |  |  |
|  | Start Time HH:MM |  |  |  |  |  |  |  |
|  | Stop Time HH:MM |  |  |  |  |  |  |  |
|  | Arm Time Period |  |  |  |  |  |  |  |
|  | Start Time HH:MM |  |  |  |  |  |  |  |
|  | Stop Time HH:MM |  |  |  |  |  |  |  |

NOTES

## VISTA-120IT Summary of Connections Diagram