COMPLIANCE NOTICE

INSTALLATION REQUIREMENTS FOR EN50131-1 COMPLIANCE

This control is EN50131-1 compliant; Security Grade 2, Environmental Class 2

Revision Level: Control microprocessor version 3.0 or higher

This document describes the required program settings that are needed to comply with the EN50131-1 specification.

Programming Field Options

The following fields must be programmed as described.

| Field | Title | Entry | Description |
| --- | --- | --- | --- |
| *22 | RF Options | 1, 2 | (If RF devices used) Selects RF Jam detect and supervision times of 20 minutes while disarmed and 2 hours while armed. |
| *26 | Chime by Zone / #65 Prog Mode | Entry 2: 1 | Requires end user to authorize Installer. Must use #65 command to allow installer to access system by master user code. |
| *35, *36 | Entry Delay 1 and 2 | Less than 45 | Entry times may not exceed 45 Seconds. |
| *50 | Dialer delay | 2 | Must be 30-second dial delay for burglary zones. |
| *56 | Zone Programming: Loop Response | 1, 2, or 3 | Loop response time entry must be either 1 (400ms); 2 (700ms); or 3 (1.2Sec). |
| *88 | Exit Options | 0 or 1 | To prevent arming if zones outside the exit route are not ready. Note that only zones in the exit route may be placed in zone list 12 if the value 1 is selected. |
| *92 | Telecom Monitor Enable | Entry 1: 1 Entry 2: 1 or 2 | Enables phone fail for one minute with keypad display. |
| *173 | RF Reporting Options | 3 | Report RF low battery and tamper while disarmed. |
| *176 | Siren Options | Entry 2 = 0 | No Siren Delay. |
| *199 | ECP Device Fail Display | 1 | Display ECP device failures. |

Hardware Requirements

1.	The red Bell Supervision jumper must be cut to enable bell supervision.

2.	The panel enclosure must be tamper protected.

3.	Any device connected to the panel outside the box must be tamper protected.

4.	The battery should be 7ah to meet the standby requirements.

165 Eileen Way, Syosset, NY  11791

Copyright © 2005 Honeywell International Inc.

www.honeywell.com/security

#### ÊK0749XXENEŠ

K0749XXEN  5/05   Rev. A